    - C-0002

Code

C-0002

Classification

Main body: Communication abnormality

Cause

Communication error between the printer control board (PRCB) and the conveyance drive board (CDB).

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Conveyance drive board (CDB)

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Deactivate the sub power switch and unplug the power plug. Restart the main body after 10 seconds.

- Check the connection status of the PRCB connector, and repair it when there is any abnormality.

- Check the connection status of the CDB connector, and repair it when there is any abnormality.

- Replace PRCB.

- Replace CDB.

Faulty part isolation DIPSW Control while detached

    - C-0101

Code

C-0101

Classification

Main body: Paper feed motor abnormality

Cause

When the paper feed motor/Up (M11) is active, an error detection signal of M11 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- DC power supply/2 (DCPS2)

- Printer control board (PRCB)

- Paper feed motor/Up (M11)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring among the motor, the PRCB, and the DCPS2. Then repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair it when there is any abnormality. (Wiring diagram: Main body (2/4):3- O)

- Replace M11.

- Replace PRCB.

- Replace DCPS2.

Faulty part isolation DIPSW Control while detached

    - C-0233

Code

C-0233

Classification

Main body: Motor abnormality

Cause

Within a specified period of time after the conveyance roller pressure release motor/2 (M36) starts returning to the home position, the conveyance roller pressure release home sensor/2 (PS16) does not become active.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Conveyance drive board (CDB)

- Printer control board (PRCB)

- Conveyance roller pressure release motor/2 (M36)

- Conveyance roller pressure release home sensor/2 (PS16)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the front door is closed properly, and repair it when there is any abnormality.

- Check the conveyance section, and repair it when there is any abnormality.

- Check the drive gear system in the conveyance section, and repair it when there is any abnormality.

- Check the connector connection and the wiring between CDB and the motor the sensor, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and CDB, and repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (1/4): 16-L)

- Check I/O and the operation of the sensor, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (1/4): 11-P)

- Replace PS16.

- Replace M36.

- Replace CDB.

- Replace PRCB.

- Replace M36 and CDB at the same time.

Faulty part isolation DIPSW Control while detached

    - C-0234

Code

C-0234

Classification

Main body: Motor abnormality

Cause

Within a specified period of time after the loop roller pressure release motor (M34) starts returning to the home position, the loop roller home sen- sor (PS17) does not become active.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Conveyance drive board (CDB)

- Printer control board (PRCB)

- Loop roller pressure release motor (M34)

- Loop roller pressure release home sensor (PS17)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the front door is closed properly, and repair it when there is any abnormality.

- Check the conveyance section, and repair it when there is any abnormality.

- Check the drive gear system in the conveyance section, and repair it when there is any abnormality.

- Check the connector connection and the wiring among the motor, the sensor and the CDB, and repair it when there is any abnormality.

- Check the connector connection and the wiring between CDB and PRCB, and repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 16-L)

- Check I/O and the operation of the sensor, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 11-P)

- Replace PS17.

- Replace M37.

- Replace CDB.

- Replace PRCB.

- Replace M37 and CDB at the same time.

Faulty part isolation DIPSW Control while detached

    - C-0238

Code

C-0238

Classification

Main body: Paper exit unit placement abnormality

Cause

The fusing unit is not set.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Paper exit unit drawer connector

- Printer control board (PRCB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the fusing unit is properly set and repair it if any abnormality.

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it if any abnormality.

- Check the connector connection and the wiring on PRCB, and repair it if any abnormality.

- Replace the fusing unit.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

  - Solution 2 (C-0301_0400)

    - C-0301*

Code

C-0301*

Classification

Main body: Fan abnormality

Cause

When the cyclone exhaust fan (FM22) is active, an error detection signal of FM22 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Cyclone exhaust fan (FM22)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between PRCB and the fun, and repair it when there is any abnormality.

- Rotate the fan or I/O manually, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (1/4): 20-F)

- Replace FM22.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-0302*

Code

C-0302*

Classification

Main body: Fan abnormality

Cause

When the paper exit cooling fan/1 (FM23) is active, an error detection signal of FM23 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Paper exit cooling fan/1 (FM23)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between PRCB and the fan, and repair it when there is any abnormality.

- Rotate the fan or I/O manually, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (1/4):22-L)

- Replace FM23.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-0304*

Code

C-0304*

Classification

Main body: Fan abnormality

Cause

When the tacking fan/1 (FM19) is active, an error detection signal of FM19 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Tacking fan/1 (FM19)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between PRCB and the fan, and repair it when there is any abnormality.

- Rotate the fan or I/O manually, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (1/4): 20-L)

- Replace FM19.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-0323*

Code

C-0323*

Classification

Main body: Fan abnormality

Cause

When the tacking fan/2 (FM20) is active, an error detection signal of FM20 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Tacking fan/2 (FM20)

- Printer control board (PRCB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between PRCB and the fun, and repair it when there is any abnormality.

- Rotate the fan manually or using I/O, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (1/4): 19-L)

- Replace FM20.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-0324*

Code

C-0324*

Classification

Main body: Fan abnormality

Cause

When the tacking fan/3 (FM21) is active, an error detection signal of FM21 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Tacking fan/3 (FM21)

- Printer control board (PRCB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between PRCB and the fun, and repair it when there is any abnormality.

- Rotate the fan manually or using I/O, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (1/4): 19-L)

- Replace FM21.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-0373*

Code

C-0373*

Classification

Main body: Fan abnormality

Cause

An error detection signal of FM24 is detected continuously for a specified period of time while the ADU exhaust fan (FM24) is active.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- ADU exhaust fan (FM24)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between PRCB and the fan, and repair it when there is any abnormality.

- Rotate the fan by I/O or manually, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (1/4): 21-L)

- Replace FM24 and PRCB at a time

Faulty part isolation DIPSW Control while detached

    - C-0374*

Code

C-0374*

Classification

Main body: Fan abnormality

Cause

When the paper exit cooling fan/2 (FM25) is active, an error detection signal of FM25 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Paper exit fan/2 (FM25)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between PRCB and the fan, and repair it when there is any abnormality.

- Rotate the fan by I/O or manually, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (1/4): 21-L)

- Replace FM25.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

  - Solution 3 (C-2001_2400)

    - C-2001

Code

C-2001

Classification

Main body: Communication abnormality

Cause

The communication error between the printer control board (PRCB) and the drum motor/Y (M1) or drum motor/M (M2). Alternatively, a checksum error is detected.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Drum motor/Y (M1)

- Drum motor/M (M2)

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Deactivate the sub power switch and unplug the power plug. Restart the main body after 10 seconds.

- Check the connector connection and the wiring between PRCB and the motor, and repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4):15-O, 16-O)

- Replace M1 or M2.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2004

Code

C-2004

Classification

Main body: Communication abnormality

Cause

The communication error between the printer control board (PRCB) and the drum motor/Y (M1) or drum motor/M (M2).

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Drum motor/Y (M1)

- Drum motor/M (M2)

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Deactivate the sub power switch and unplug the power plug. Restart the main body after 10 seconds.

- Check the connector connection and the wiring between PRCB and the motor, and repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4):15-O, 16-O)

- Replace M1 or M2.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2201

Code

C-2201

Classification

Main body: Developing motor abnormality

Cause

When the developing motor/Y (M8) is active, an error detection signal of M8 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing motor/Y (M8)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between PRCB and the motor, and repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4):16-B)

- Replace M8 and PRCB. Faulty part isolation DIPSW Control while detached

    - C-2202

Code

C-2202

Classification

Main body: Developing motor abnormality

Cause

When the developing motor/M (M7) is active, an error detection signal of M7 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing motor/M (M7)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between PRCB and the motor, and repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4):16-B)

- Replace M7 and PRCB. Faulty part isolation DIPSW Control while detached

    - C-2203

Code

C-2203

Classification

Main body: Developing motor abnormality

Cause

When the developing motor/C (M6) is active, an error detection signal of M6 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing motor/C (M6)

Correction Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between PRCB and the motor, and repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4):15-B)

- Replace M6 and PRCB. Faulty part isolation DIPSW Control while detached

    - C-2204

Code

C-2204

Classification

Main body: Developing motor abnormality

Cause

When the developing motor/K (M5) is active, An error detection signal of M5 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing motor/K (M5)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between PRCB and the motor, and repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4):15-B)

- Replace M5 and PRCB. Faulty part isolation DIPSW Control while detached

    - C-2211

Code

C-2211

Classification

Main body: Drum motor abnormality

Cause

When the drum motor/Y (M1) is active, an error detection signal of M1 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Drum motor/Y (M1)

- Encoder sensor/Y1 (PS52), Encoder sensor/Y2 (PS53)

- DC power supply/2 (DCPS2)

- Drum unit/Y

- Drum encoder/Y

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the drum unit and repair or replace it when there is any abnormality

- Check the connector connection and the wiring between PRCB, and the motor and the sensor, and repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 15-O)

- Check the connector connection and the wiring between DCPS2 and PRCB, and repair it when there is any abnormality.

- Check the skew, the status of the slit, damage, dust of the drum encoder. Repair or replace it when there is any abnormality.

- Replace PS52 and PS53.

- Replace M1.

- Replace PRCB.

- Replace DCPS2.

Faulty part isolation DIPSW Control while detached

    - C-2212

Code

C-2212

Classification

Main body: Drum motor abnormality

Cause

When the drum motor/M (M2) is active, an error detection signal of M15 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Drum motor/M (M2)

- Encoder sensor/M1 (PS54), Encoder sensor/M2 (PS55)

- DC power supply/2 (DCPS2)

- Drum unit/M

- Drum encoder/M

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the drum unit and repair or replace it when there is any abnormality

- Check the connector connection and the wiring between PRCB, and the motor and the sensor, and repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 16-O)

- Check the connector connection and the wiring between DCPS2 and PRCB, and repair it when there is any abnormality.

- Check the skew, the status of the slit, damage, dust of the drum encoder. Repair or replace it when there is any abnormality.

- Replace PS54 and PS55.

- Replace M2.

- Replace PRCB.

- Replace DCPS2.

Faulty part isolation DIPSW Control while detached

    - C-2213

Code

C-2213

Classification

Main body: Drum motor abnormality

Cause

When the drum motor/C (M3) is active, an error detection signal of M3 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Drum motor/C (M3)

- Encoder sensor/C1 (PS57), encoder sensor/C2 (PS56)

- DC power supply/2 (DCPS2)

- Drum unit/C

- Drum encoder/C

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the drum unit and repair or replace it when there is any abnormality

- Check the connector connection and the wiring between PRCB, and the motor and the sensor, and repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 17-O)

- Check the connector connection and the wiring between DCPS2 and PRCB, and repair it when there is any abnormality.

- Check the skew, the status of the slit, damage, dust of the drum encoder. Repair or replace it when there is any abnormality.

- Replace PS56 and PS57.

- Replace M3.

- Replace PRCB.

- Replace DCPS2.

Faulty part isolation DIPSW Control while detached

    - C-2214

Code

C-2214

Classification

Main body: Drum motor abnormality

Cause

When the drum motor/K (M4) is active, an error detection signal of M4 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Drum motor/K (M4)

- Encoder sensor/K1 (EN58), encoder sensor/K2 (EN59)

- DC power supply/2 (DCPS2)

- Drum unit/K

- Drum drive board (DDB)

- Drum encoder/K

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the drum unit and repair or replace it when there is any abnormality

- Check the connector connection and the wiring between PRCB, and the motor and the sensor, and repair it when there is any abnormality.

- Check the connector connection and the wiring between DDB and the motor, DDB and the PRCB and repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 17-O)

- Check the connector connection and the wiring between DCPS2 and PRCB, and repair it when there is any abnormality.

- Check the skew, the status of the slit, damage, dust of the drum encoder. Repair or replace it when there is any abnormality.

- Replace EN58, EN59.

- Replace M4.

- Replace PRCB.

- Replace DCPS2.

Faulty part isolation DIPSW Control while detached

    - C-2220

Code

C-2220

Classification

Main body: Transfer belt unit abnormality

Cause

When the intermediate transfer motor (M9) is active, an error detection signal of M9 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Intermediate transfer motor (M9)

- Intermediate transfer belt encoder sensor (PS49)

- DC power supply/2 (DCPS2)

- Intermediate transfer unit

- Intermediate transfer belt encoder

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the intermediate transfer unit and repair and replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB, the motor and sensor, and repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4):21-D)

- Check the connector connection and the wiring between DCPS2 and PRCB, and repair it when there is any abnormality.

- Check the skew, the status of the slit, damage, dust of the intermediate transfer belt encoder. Repair or replace it when there is any abnormal- ity.

- Replace M9.

- Replace PS49.

- Replace DCPS2.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2221

Code

C-2221

Classification

Main body: 1st transfer belt motor abnormality

Cause

When either of the 1st transfer pressure sensor/1 (PS50) or the 1st transfer pressure sensor/2 (PS51) is active: Within a specified period of time after the 1st transfer pressure release motor (M10) becomes active, either of PS50 or PS51 is not deactivated.

When PS50 or PS51 is not active: Within a specified period of time after M10 becomes active, PS50 or PS51 does not become active.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- DC power supply/1 (DCPS1)

- Printer control board (PRCB)

- 1st transfer pressure release motor (M10)

- 1st transfer pressure sensor/1 (PS50)

- 1st transfer pressure sensor/2 (PS51)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the 1st transfer fusing pressure release section, and clean or repair it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the sensor, and repair it when there is any abnormality.

- Check the connector connection and the wiring between DCPS1 and the motor, and repair it when there is any abnormality.

- Check the connector connection and the wiring between DCPS1 and PRCB, and repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4):22-D)

- Check I/O and the operation of the sensor, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4):23-D)

- Replace PS50.

- Replace PS51.

- Replace M10.

- Replace DCPS1.

- Replace PRCB.

- Replace M10 and DCPS1 at a time.

Faulty part isolation DIPSW Control while detached

    - C-2222

Code

C-2222

Classification

Main body: Toner bottle motor abnormality

Cause

When the toner bottle motor (M15) is active, an error detection signal is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Toner bottle motor (M15)

- DC power supply/2 (DCPS2)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the toner bottle door is closed properly, and repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4):12-D)

- Check the connector connection and the wiring between DCPS2 and the motor, and repair it when there is any abnormality.

- Check the connector connection and the wiring between DCPS2 and PRCB, and repair it when there is any abnormality.

- Replace M15.

- Replace PRCB.

- Replace DCPS2.

Faulty part isolation DIPSW Control while detached

    - C-2223

Code

C-2223

Classification

Main body: Low drum load torque abnormality

Cause

Within a specified period of time after the low load torque recovery control of the drum motor/Y (M1), the drum motor/M (M2), the drum motor/C (M3), the drum motor/K (M4) starts, either of the torque is not recovered.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Drum motor/Y (M1)

- Drum motor/M (M2)

- Drum motor/C (M3)

- Drum motor/K (M4)

- Printer control board (PRCB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the drum unit, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the motor, and repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4):15-O, 16-O, 17-O)

- Replace M1, M2, M3 or M4.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2224

Code

C-2224

Classification

Main body: Toner collection motor/Up

Cause

When the waste toner collection motor/1 (M13) is active, an error detection signal of M13 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Toner collection motor/1 (M13)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the toner supply section, and repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 4-B)

- Replace M13.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2225

Code

C-2225

Classification

Main body: Toner collection motor/Down

Cause

When the waste toner collection motor/2 (M14) is active, an error detection signal of M14 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Toner collection motor/2 (M14)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the toner supply section, and repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 5-B)

- Replace M14.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2302*

Code

C-2302*

Classification

Main body: Fan abnormality

Cause

When the drum motor cooling fan/1 (FM6) is active, an error detection signal of FM6 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Drum motor cooling fan/1 (FM6)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between PRCB and the fun, and repair it when there is any abnormality.

- Rotate the fan or I/O manually, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (1/4): 10-E)

- Replace FM6.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2303*

Code

C-2303*

Classification

Main body: Fan abnormality

Cause

When the transfer belt fan (FM17) is active, an error detection signal of FM17 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Transfer belt fan (FM17)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between PRCB and the fun, and repair it when there is any abnormality.

- Rotate the fan or I/O manually, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (1/4): 14-L)

- Replace FM17.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2304*

Code

C-2304*

Classification

Main body: Fan abnormality

Cause

When the developing fan/1 (FM26) is active, an error detection signal of FM26 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing fan/1 (FM26)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between PRCB and the fun, and repair it when there is any abnormality.

- Rotate the fan or I/O manually, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4):2-B)

- Replace FM26 and PRCB at a time.

Faulty part isolation DIPSW Control while detached

    - C-2305*

Code

C-2305*

Classification

Main body: Fan abnormality

Cause

When the developing fan/2 (FM27) is active, an error detection signal of FM27 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing fan/2 (FM27)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between PRCB and the fun, and repair it when there is any abnormality.

- Rotate the fan or I/O manually, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4):3-B)

- Replace FM27.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2306*

Code

C-2306*

Classification

Main body: Fan abnormality

Cause

When the drum motor cooling fan/2 (FM7) is active, an error detection signal of FM7 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Drum motor cooling fan/2 (FM7)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between PRCB and the fun, and repair it when there is any abnormality.

- Rotate the fan or I/O manually, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (1/4): 10-E)

- Replace FM7.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2307*

Code

C-2307*

Classification

Main body: Fan abnormality

Cause

When the developing fan/3 (FM28) is active, an error detection signal of FM28 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing fan/3 (FM28)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between PRCB and the fan, and repair it when there is any abnormality.

- Rotate the fan by I/O or manually, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 3-B)

- Replace FM28.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

  - Solution 4 (C-2401_2500)

    - C-2401

Code

C-2401

Classification

Main body: Erase lamp abnormality

Cause

It cannot be detected that the erase lamp/Y (EL1) is installed.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Erase lamp/Y (EL1)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check the EL section, and clean or repair it when there is any abnormality. (Wiring diagram: Main body (2/4):17-B)

- Check the connector connection and the wiring between PRCB and EL, and repair it when there is any abnormality.

- Replace EL.

- Replace PRCB.

- Replace EL and PRCB at a time. Faulty part isolation DIPSW Control while detached

    - C-2402

Code

C-2402

Classification

Main body: Erase lamp abnormality

Cause

It cannot be detected that the erase lamp/M (EL2) is installed.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Erase lamp/M (EL2)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check the EL section, and clean or repair it when there is any abnormality. (Wiring diagram: Main body (2/4):18-B)

- Check the connector connection and the wiring between PRCB and EL, and repair it when there is any abnormality.

- Replace EL.

- Replace PRCB.

- Replace EL and PRCB at a time. Faulty part isolation DIPSW Control while detached

    - C-2403

Code

C-2403

Classification

Main body: Erase lamp abnormality

Cause

It cannot be detected that the erase lamp/C (EL4) is installed.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Erase lamp/C (EL4)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check the EL section, and clean or repair it when there is any abnormality. (Wiring diagram: Main body (2/4):18-B)

- Check the connector connection and the wiring between PRCB and EL, and repair it when there is any abnormality.

- Replace EL.

- Replace PRCB.

- Replace EL and PRCB at a time. Faulty part isolation DIPSW Control while detached

    - C-2404

Code

C-2404

Classification

Main body: Erase lamp abnormality

Cause

It cannot be detected that the erase lamp/K (EL3) is installed.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Erase lamp/K (EL3)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check the EL section, and clean or repair it when there is any abnormality. (Wiring diagram: Main body (2/4):18-B)

- Check the connector connection and the wiring between PRCB and EL, and repair it when there is any abnormality.

- Replace EL.

- Replace PRCB.

- Replace EL and PRCB at a time. Faulty part isolation DIPSW Control while detached

    - C-2431

Code

C-2431

Classification

Main body: TCR sensor initial adjustment abnormality (low density)

Cause

When the initial adjustment is performed with the TCR sensor/Y (TCRS1), the output value of TCRS1 lowers the prescribed value.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing motor/Y (M8)

- TCR sensor/Y (TCRS1)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check TCRS, and clean or repair it when there is a sensor dirt or any abnormality. (Wiring diagram: Main body (2/4):19-B)

- Check the developing section, and clean or repair it if there is a sensor dirt or any abnormality.

- Check the connector connection and the wiring between PRCB and TCRS, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the developing motor, and repair it when there is any abnormality.

- Rotate the developing motor by I/O, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 16-B)

- Replace TCRS.

- Replace M8.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2432

Code

C-2432

Classification

Main body: TCR sensor initial adjustment abnormality (low density)

Cause

When the initial adjustment is performed with the TCR sensor/M (TCRS2), the output value of TCRS2 lowers the prescribed value.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing motor/M (M7)

- TCR sensor/M (TCRS2)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check TCRS, and clean or repair it when there is a sensor dirt or any abnormality. (Wiring diagram: Main body (2/4):19-B)

- Check the developing section, and clean or repair it if there is a sensor dirt or any abnormality.

- Check the connector connection and the wiring between PRCB and TCRS, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the developing motor, and repair it when there is any abnormality.

- Rotate the developing motor by I/O, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 16-B)

- Replace TCRS.

- Replace M7.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2433

Code

C-2433

Classification

Main body: TCR sensor initial adjustment abnormality (low density)

Cause

When the initial adjustment is performed with the TCR sensor/C (TCRS4), the output value of TCRS4 lowers the prescribed value.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing motor/C (M6)

- TCR sensor/C (TCRS4)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check TCRS, and clean or repair it when there is a sensor dirt or any abnormality. (Wiring diagram: Main body (2/4):20-B)

- Check the developing section, and clean or repair it if there is a sensor dirt or any abnormality.

- Check the connector connection and the wiring between PRCB and TCRS, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the developing motor, and repair it when there is any abnormality.

- Rotate the developing motor by I/O, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 15-B)

- Replace TCRS.

- Replace M6.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2434

Code

C-2434

Classification

Main body: TCR sensor initial adjustment abnormality (low density)

Cause

When the initial adjustment is performed with the TCR sensor/K (TCRS3), the output value of TCRS3 lowers the prescribed value.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing motor/K (M5)

- TCR sensor/K (TCRS3)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check TCRS, and clean or repair it when there is a sensor dirt or any abnormality. (Wiring diagram: Main body (2/4):20-B)

- Check the developing section, and clean or repair it if there is a sensor dirt or any abnormality.

- Check the connector connection and the wiring between PRCB and TCRS, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the developing motor, and repair it when there is any abnormality.

- Rotate the developing motor by I/O, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 15-B)

- Replace TCRS.

- Replace M5.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2441

Code

C-2441

Classification

Main body: TCR sensor initial adjustment abnormality (high density)

Cause

When the initial adjustment is performed with the TCR sensor/Y (TCRS1), the output value of TCRS1 exceeds the prescribed value.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing motor/Y (M8)

- TCR sensor/Y (TCRS1)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check TCRS, and clean or repair it when there is a sensor dirt or any abnormality. (Wiring diagram: Main body (2/4):19-B)

- Check the developing section, and clean or repair it if there is a sensor dirt or any abnormality.

- Check the connector connection and the wiring between PRCB and TCRS, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the developing motor, and repair it when there is any abnormality.

- Rotate the developing motor by I/O, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 16-B)

- Replace TCRS.

- Replace M8.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2442

Code

C-2442

Classification

Main body: TCR sensor initial adjustment abnormality (high density)

Cause

When the initial adjustment is performed with the TCR sensor/M (TCRS2), the output value of TCRS2 exceeds the prescribed value.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing motor/M (M7)

- TCR sensor/M (TCRS2)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check TCRS, and clean or repair it when there is a sensor dirt or any abnormality. (Wiring diagram: Main body (2/4):19-B)

- Check the developing section, and clean or repair it if there is a sensor dirt or any abnormality.

- Check the connector connection and the wiring between PRCB and TCRS, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the developing motor, and repair it when there is any abnormality.

- Rotate the developing motor by I/O, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 16-B)

- Replace TCRS.

- Replace M7.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2443

Code

C-2443

Classification

Main body: TCR sensor initial adjustment abnormality (high density)

Cause

When the initial adjustment is performed with the TCR sensor/C (TCRS4), the output value of TCRS4 exceeds the prescribed value.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing motor/C (M6)

- TCR sensor/C (TCRS4)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check TCRS, and clean or repair it when there is a sensor dirt or any abnormality. (Wiring diagram: Main body (2/4):20-B)

- Check the developing section, and clean or repair it if there is a sensor dirt or any abnormality.

- Check the connector connection and the wiring between PRCB and TCRS, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the developing motor, and repair it when there is any abnormality.

- Rotate the developing motor by I/O, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 15-B)

- Replace TCRS.

- Replace M6.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2444

Code

C-2444

Classification

Main body: TCR sensor initial adjustment abnormality (high density)

Cause

When the initial adjustment is performed with the TCR sensor/K (TCRS3), the output value of TCRS3 exceeds the prescribed value.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing motor/K (M5)

- TCR sensor/K (TCRS3)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check TCRS, and clean or repair it when there is a sensor dirt or any abnormality. (Wiring diagram: Main body (2/4):20-B)

- Check the developing section, and clean or repair it if there is a sensor dirt or any abnormality.

- Check the connector connection and the wiring between PRCB and TCRS, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the developing motor, and repair it when there is any abnormality.

- Rotate the developing motor by I/O, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 15-B)

- Replace TCRS.

- Replace M5.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2451

Code

C-2451

Classification

Main body: Toner density abnormality (Light)

Cause

Under the control of the TCR sensor/Y (TCRS1), TCRS1 detects the toner density that is lighter than the prescribed value for a longer period than the specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing motor/Y (M8)

- TCR sensor/Y (TCRS1)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check TCRS, and clean or repair it when there is a sensor dirt or any abnormality. (Wiring diagram: Main body (2/4):19-B)

- Check the developing section, and clean or repair it if there is a sensor dirt or any abnormality.

- Check the connector connection and the wiring between PRCB and TCRS, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the developing motor, and repair it when there is any abnormality.

- Rotate the developing motor by I/O, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 16-B)

- Replace TCRS.

- Replace M8.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2452

Code

C-2452

Classification

Main body: Toner density abnormality (Light)

Cause

Under the control of the TCR sensor/M (TCRS2), TCRS2 detects the toner density that is lighter than the prescribed value for a longer period than the specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing motor/M (M7)

- TCR sensor/M (TCRS2)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check TCRS, and clean or repair it when there is a sensor dirt or any abnormality. (Wiring diagram: Main body (2/4):19-B)

- Check the developing section, and clean or repair it if there is a sensor dirt or any abnormality.

- Check the connector connection and the wiring between PRCB and TCRS, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the developing motor, and repair it when there is any abnormality.

- Rotate the developing motor by I/O, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 16-B)

- Replace TCRS.

- Replace M7.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2453

Code

C-2453

Classification

Main body: Toner density abnormality (Light)

Cause

Under the control of the TCR sensor/C (TCRS4), TCRS4 detects the toner density that lowers the prescribed value for a longer period than the specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing motor/C (M6)

- TCR sensor/C (TCRS4)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check TCRS, and clean or repair it when there is a sensor dirt or any abnormality. (Wiring diagram: Main body (2/4):20-B)

- Check the developing section, and clean or repair it if there is a sensor dirt or any abnormality.

- Check the connector connection and the wiring between PRCB and TCRS, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the developing motor, and repair it when there is any abnormality.

- Rotate the developing motor by I/O, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 15-B)

- Replace TCRS.

- Replace M6.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2454

Code

C-2454

Classification

Main body: Toner density abnormality (Light)

Cause

Under the control of the TCR sensor/K (TCRS3), TCRS3 detects the toner density that is lighter than the prescribed value for a longer period than the specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing motor/K (M5)

- TCR sensor/K (TCRS3)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check TCRS, and clean or repair it when there is a sensor dirt or any abnormality. (Wiring diagram: Main body (2/4):20-B)

- Check the developing section, and clean or repair it if there is a sensor dirt or any abnormality.

- Check the connector connection and the wiring between PRCB and TCRS, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the developing motor, and repair it when there is any abnormality.

- Rotate the developing motor by I/O, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 15-B)

- Replace TCRS.

- Replace M5.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2455

Code

C-2455

Classification

Main body: Toner density abnormality (Shadow)

Cause

Under the control of the TCR sensor/Y (TCRS1), TCRS1 detects the toner density that is darker than the prescribed value for a longer period than the specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing motor/Y (M8)

- TCR sensor/Y (TCRS1)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check TCRS, and clean or repair it when there is a sensor dirt or any abnormality. (Wiring diagram: Main body (2/4): 19-B)

- Check the developing section, and clean or repair it when there is a sensor dirt or any abnormality.

- Check the connector connection and the wiring between PRCB and TCRS, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the developing motor, and repair it when there is any abnormality.

- Rotate the developing motor by I/O, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 16-B)

- Replace TCRS.

- Replace M8.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2456

Code

C-2456

Classification

Main body: Toner density abnormality (Shadow)

Cause

Under the control of the TCR sensor/M (TCRS2), TCRS2 detects the toner density that is darker than the prescribed value for a longer period than the specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing motor/M (M7)

- TCR sensor/M (TCRS2)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check TCRS, and clean or repair it when there is a sensor dirt or any abnormality. (Wiring diagram: Main body (2/4): 19-B)

- Check the developing section, and clean or repair it when there is a sensor dirt or any abnormality.

- Check the connector connection and the wiring between PRCB and TCRS, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the developing motor, and repair it when there is any abnormality.

- Rotate the developing motor by I/O, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 16-B)

- Replace TCRS.

- Replace M7.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2457

Code

C-2457

Classification

Main body: Toner density abnormality (Shadow)

Cause

Under the control of the TCR sensor/C (TCRS4), TCRS4 detects the toner density that is darker than the prescribed value for a longer period than the specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing motor/C (M6)

- TCR sensor/C (TCRS4)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check TCRS, and clean or repair it when there is a sensor dirt or any abnormality. (Wiring diagram: Main body (2/4): 20-B)

- Check the developing section, and clean or repair it when there is a sensor dirt or any abnormality.

- Check the connector connection and the wiring between PRCB and TCRS, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the developing motor, and repair it when there is any abnormality.

- Rotate the developing motor by I/O, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 15-B)

- Replace TCRS.

- Replace M6.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2458

Code

C-2458

Classification

Main body: Toner density abnormality (Shadow)

Cause

Under the control of the TCR sensor/K (TCRS3), TCRS3 detects the toner density that is darker than the prescribed value for a longer period than the specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing motor/K (M5)

- TCR sensor/K (TCRS3)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check TCRS, and clean or repair it when there is a sensor dirt or any abnormality. (Wiring diagram: Main body (2/4): 20-B)

- Check the developing section, and clean or repair it when there is a sensor dirt or any abnormality.

- Check the connector connection and the wiring between PRCB and TCRS, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the developing motor, and repair it when there is any abnormality.

- Rotate the developing motor by I/O, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 15-B)

- Replace TCRS.

- Replace M5.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2470

Code

C-2470

Classification

Main body: Process unit mount connection abnormality

Cause

The process unit mount is not connected.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Process mount

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check the process mount, and repair or replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the process mount, and repair it when there is any abnormality.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

  - Solution 5 (C-2701_2800)

    - C-2701

Code

C-2701

Classification

Main body: High voltage unit/1 abnormality

Cause

An error detection signal of the charging corona/Y is detected while the charging corona/Y is activated. The signal is detected even after you acti- vate and deactivate the charging corona/Y for a prescribed time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- High voltage unit/1 (HV1)

- Charging corona/Y

- Printer control board (PRCB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection of the high voltage contact of the process unit, and clean or repair it when there is any abnormality.

- Check the charger (charging wire and the grid), and clean or repair it if there is any dirt or abnormality.

- Check the connector connection and the wiring between HV1 and the charger, and repair it when there is any abnormality. (Wiring diagram: Main body (3/4): 1-K)

- Check the connector connection and the wiring between PRCB and HV1, and repair it when there is any abnormality. (Wiring diagram: Main body (3/4): 1-K).

- Replace HV1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2702

Code

C-2702

Classification

Main body: High voltage unit/1 abnormality

Cause

An error detection signal of the charging corona/M is detected while the charging corona/M is activated. The signal is detected even after you acti- vate and deactivate the charging corona/M for a prescribed time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- High voltage unit/1 (HV1)

- Charging corona/M

- Printer control board (PRCB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection of the high voltage contact of the process unit, and clean or repair it when there is any abnormality.

- Check the charger (charging wire and the grid), and clean or repair it if there is any dirt or abnormality.

- Check the connector connection and the wiring between HV1 and the charger, and repair it when there is any abnormality. (Wiring diagram: Main body (3/4): 1-K)

- Check the connector connection and the wiring between PRCB and HV1, and repair it when there is any abnormality. (Wiring diagram: Main body (3/4): 1-K).

- Replace HV1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2703

Code

C-2703

Classification

Main body: High voltage unit/1 abnormality

Cause

An error detection signal of the charging corona/C is detected while the charging corona/C is active. The signal is detected even after you activate and deactivate the charging corona/C for a prescribed time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- High voltage unit/1 (HV1)

- Charging corona/C

- Printer control board (PRCB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection of the high voltage contact of the process unit, and clean or repair it when there is any abnormality.

- Check the charger (charging wire and the grid), and clean or repair it if there is any dirt or abnormality.

- Check the connector connection and the wiring between HV1 and the charger, and repair it when there is any abnormality. (Wiring diagram: Main body (3/4): 1-K)

- Check the connector connection and the wiring between PRCB and HV1, and repair it when there is any abnormality. (Wiring diagram: Main body (3/4): 1-K).

- Replace HV1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2704

Code

C-2704

Classification

Main body: High voltage unit/1 abnormality

Cause

An error detection signal of the charging corona/K is detected while the charging corona/K is active. The signal is detected even after you activate and deactivate the charging corona/K for a prescribed time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- High voltage unit/1 (HV1)

- Charging corona/K

- Printer control board (PRCB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection of the high voltage contact of the process unit, and clean or repair it when there is any abnormality.

- Check the charger (charging wire and the grid), and clean or repair it if there is any dirt or abnormality.

- Check the connector connection and the wiring between HV1 and the charger, and repair it when there is any abnormality. (Wiring diagram: Main body (3/4): 1-K)

- Check the connector connection and the wiring between PRCB and HV1, and repair it when there is any abnormality. (Wiring diagram: Main body (3/4): 1-K).

- Replace HV1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2711

Code

C-2711

Classification

Main body: High voltage unit/2 abnormality

Cause

An error detection signal of the 1st transfer/Y is detected while the 1st transfer/Y is active. The signal is detected even after you activate and deac- tivate the 1st transfer/Y for a prescribed time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- High voltage unit/2 (HV2)

- 1st transfer roller/Y

- Printer control board (PRCB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection of the high voltage contact of the process unit, and clean or repair it when there is any abnormality.

- Check the connector connection and the wiring between HV2 and the 1st transfer unit, and repair it when there is any abnormality. (Wiring diagram: Main body (3/4): 5-J)

- Check the connector connection and the wiring between PRCB and HV2, and repair it when there is any abnormality.

- Replace HV2.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2712

Code

C-2712

Classification

Main body: High voltage unit/2 abnormality

Cause

An error detection signal of the 1st transfer/M is detected while the 1st transfer/M is active. The signal is detected even after you activate and deactivate the 1st transfer/M for a prescribed time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- High voltage unit/2 (HV2)

- 1st transfer roller/M

- Printer control board (PRCB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection of the high voltage contact of the process unit, and clean or repair it when there is any abnormality.

- Check the connector connection and the wiring between HV2 and the 1st transfer unit, and repair it when there is any abnormality. (Wiring diagram: Main body (3/4): 5-J)

- Check the connector connection and the wiring between PRCB and HV2, and repair it when there is any abnormality.

- Replace HV2.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2713

Code

C-2713

Classification

Main body: High voltage unit/2 abnormality

Cause

An error detection signal of the 1st transfer C is detected while the 1st transfer/C is active. The signal is detected even after you deactivate and activate the 1st transfer C for a prescribed time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- High voltage unit/2 (HV2)

- 1st transfer roller/C

- Printer control board (PRCB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection of the high voltage contact of the process unit, and clean or repair it when there is any abnormality.

- Check the connector connection and the wiring between HV2 and the 1st transfer unit, and repair it when there is any abnormality. (Wiring diagram: Main body (3/4): 5-J)

- Check the connector connection and the wiring between PRCB and HV2, and repair it when there is any abnormality.

- Replace HV2.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2714

Code

C-2714

Classification

Main body: High voltage unit/2 abnormality

Cause

An error detection signal of the 1st transfer/K is detected while the 1st transfer/K is active. The signal is detected even after you deactivate and activate the 1st transfer/K for a prescribed time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- High voltage unit/2 (HV2)

- 1st transfer roller/K

- Printer control board (PRCB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection of the high voltage contact of the process unit, and clean or repair it when there is any abnormality.

- Check the connector connection and the wiring between HV2 and the 1st transfer unit, and repair it when there is any abnormality. (Wiring diagram: Main body (3/4): 5-J)

- Check the connector connection and the wiring between PRCB and HV2, and repair it when there is any abnormality.

- Replace HV2.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2720

Code

C-2720

Classification

Main body: High voltage unit/2 abnormality

Cause

An error detection signal of the 2nd transfer is detected while the 2nd transfer is active. The signal is detected even after you activate and deacti- vate the 2nd transfer for a prescribed time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- High voltage unit/2 (HV2)

- 2nd transfer roller

- Printer control board (PRCB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection of the high voltage contact of the process unit, and clean or repair it when there is any abnormality.

- Check the connector connection and the wiring between HV2 and the 2nd transfer unit, and repair it when there is any abnormality. (Wiring diagram: Main body (3/4): 5-J)

- Check the connector connection and the wiring between PRCB and HV2, and repair it when there is any abnormality.

- Replace HV2.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2721

Code

C-2721

Classification

Main body: High voltage unit/2 abnormality

Cause

An error detection signal of the separation charger is detected while the separation charger is activated. The signal is detected even after you acti- vate and deactivate the separation charger for a prescribed time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- High voltage unit/2 (HV2)

- Separation charger

- Printer control board (PRCB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection of the high voltage contact of the process unit, and clean or repair it when there is any abnormality.

- Check the connector connection and the wiring between HV2 and the separation charger, and repair it when there is any abnormality. (Wiring diagram: Main body (3/4): 5-J)

- Check the connector connection and the wiring between PRCB and HV2, and repair it when there is any abnormality.

- Replace HV2.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

  - Solution 6 (C-2801_3000)

    - C-2801

Code

C-2801

Classification

Main body: Gamma correction abnormality

Cause

The output voltage of the IDC sensor (IDCS) is detected more than the specified value on the patch/Y of the developing DC maximum output.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing unit/Y

- IDC sensor (IDCS)

- Color registration shutter solenoid (SD4)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check the installation of the developing unit and I/O, and repair or replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and IDCS, and repair it when there is any abnormality.

- Check the I/O of SD4 and repair or replace it when there is any abnormality.

- Check the sensor and clean or replace it when there is any dirt or abnormality. (Wiring diagram: Main body (2/4): 23-O)

- Replace IDCS.

- Replacing the developing unit

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2802

Code

C-2802

Classification

Main body: Gamma correction abnormality

Cause

The output voltage of the IDC sensor (IDCS) is detected more than the specified value on the patch/M of the developing DC maximum output.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing unit/M

- IDC sensor (IDCS)

- Color registration shutter solenoid (SD4)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check the installation of the developing unit and I/O, and repair or replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and IDCS, and repair it when there is any abnormality.

- Check the I/O of SD4 and repair or replace it when there is any abnormality.

- Check the sensor and clean or replace it when there is any dirt or abnormality. (Wiring diagram: Main body (2/4): 23-O)

- Replace IDCS.

- Replacing the developing unit

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2803

Code

C-2803

Classification

Main body: Gamma correction abnormality

Cause

The output voltage of the IDC sensor (IDCS) is detected more than the specified value on the patch/C of the developing DC maximum output.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing unit/C

- IDC sensor (IDCS)

- Color registration shutter solenoid (SD4)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check the installation of the developing unit and I/O, and repair or replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and IDCS, and repair it when there is any abnormality.

- Check the I/O of SD4 and repair or replace it when there is any abnormality.

- Check the sensor and clean or replace it when there is any dirt or abnormality. (Wiring diagram: Main body (2/4): 23-O)

- Replace IDCS.

- Replacing the developing unit

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2804

Code

C-2804

Classification

Main body: Gamma correction abnormality

Cause

The output voltage of the IDC sensor (IDCS) is detected more than the specified value on the patch/K of the developing DC maximum output.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing unit/K

- IDC sensor (IDCS)

- Color registration shutter solenoid (SD4)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check the installation of the developing unit and I/O, and repair or replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and IDCS, and repair it when there is any abnormality.

- Check the I/O of SD4 and repair or replace it when there is any abnormality.

- Check the sensor and clean or replace it when there is any dirt or abnormality. (Wiring diagram: Main body (2/4): 23-O)

- Replace IDCS.

- Replacing the developing unit

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2811

Code

C-2811

Classification

Main body: Gamma correction abnormality

Cause

The proper value cannot be figured out within the specified time by the correction of the patch density correction/Y. Alternatively, the proper value of the developing DC output is 0 or lower.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing unit/Y

- IDC sensor (IDCS)

- Color registration shutter solenoid (SD4)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check the installation of the developing unit and I/O, and repair or replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and IDCS, and repair it when there is any abnormality.

- Check the I/O of SD4 and repair or replace it when there is any abnormality.

- Check the sensor and clean or replace it when there is any dirt or abnormality. (Wiring diagram: Main body (2/4): 23-O)

- Replace IDCS.

- Replacing the developing unit

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2812

Code

C-2812

Classification

Main body: Gamma correction abnormality

Cause

The proper value cannot be figured out within the specified time by the correction of the patch density correction/M. Alternatively, the proper value of the developing DC output is 0 or lower.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing unit/M

- IDC sensor (IDCS)

- Color registration shutter solenoid (SD4)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check the installation of the developing unit and I/O, and repair or replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and IDCS, and repair it when there is any abnormality.

- Check the I/O of SD4 and repair or replace it when there is any abnormality.

- Check the sensor and clean or replace it when there is any dirt or abnormality. (Wiring diagram: Main body (2/4): 23-O)

- Replace IDCS.

- Replacing the developing unit

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2813

Code

C-2813

Classification

Main body: Gamma correction abnormality

Cause

The proper value cannot be figured out within the specified time by the correction of the patch density correction/C. Alternatively, the proper value of the developing DC output is 0 or lower.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing unit/C

- IDC sensor (IDCS)

- Color registration shutter solenoid (SD4)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check the installation of the developing unit and I/O, and repair or replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and IDCS, and repair it when there is any abnormality.

- Check the I/O of SD4 and repair or replace it when there is any abnormality.

- Check the sensor and clean or replace it when there is any dirt or abnormality. (Wiring diagram: Main body (2/4): 23-O)

- Replace IDCS.

- Replacing the developing unit

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2814

Code

C-2814

Classification

Main body: Gamma correction abnormality

Cause

The proper value cannot be figured out within the specified time by the correction of the patch density correction/K. Alternatively, the proper value of the developing DC output is 0 or lower.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing unit/K

- IDC sensor (IDCS)

- Color registration shutter solenoid (SD4)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check the installation of the developing unit and I/O, and repair or replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and IDCS, and repair it when there is any abnormality.

- Check the I/O of SD4 and repair or replace it when there is any abnormality.

- Check the sensor and clean or replace it when there is any dirt or abnormality. (Wiring diagram: Main body (2/4): 23-O)

- Replace IDCS.

- Replacing the developing unit

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2821

Code

C-2821

Classification

Main body: Dot diameter correction abnormality

Cause

The output voltage of the IDC sensor (IDCS) is detected more than the specified value on the patch/Y of the MPC maximum output.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing unit/Y

- IDC sensor (IDCS)

- Color registration shutter solenoid (SD4)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check the installation of the developing unit and I/O, and repair or replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and IDCS, and repair it when there is any abnormality.

- Check the I/O of SD4 and repair or replace it when there is any abnormality.

- Check the sensor and clean or replace it when there is any dirt or abnormality. (Wiring diagram: Main body (2/4): 23-O)

- Replace IDCS.

- Replacing the developing unit

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2822

Code

C-2822

Classification

Main body: Dot diameter correction abnormality

Cause

The output voltage of the IDC sensor (IDCS) is detected more than the specified value on the patch/M of the MPC maximum output.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing unit/M

- IDC sensor (IDCS)

- Color registration shutter solenoid (SD4)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check the installation of the developing unit and I/O, and repair or replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and IDCS, and repair it when there is any abnormality.

- Check the I/O of SD4 and repair or replace it when there is any abnormality.

- Check the sensor and clean or replace it when there is any dirt or abnormality. (Wiring diagram: Main body (2/4): 23-O)

- Replace IDCS.

- Replacing the developing unit

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2823

Code

C-2823

Classification

Main body: Dot diameter correction abnormality

Cause

The output voltage of the IDC sensor (IDCS) is detected more than the specified value on the patch/C of the MPC maximum output.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing unit/C

- IDC sensor (IDCS)

- Color registration shutter solenoid (SD4)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check the installation of the developing unit and I/O, and repair or replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and IDCS, and repair it when there is any abnormality.

- Check the I/O of SD4 and repair or replace it when there is any abnormality.

- Check the sensor and clean or replace it when there is any dirt or abnormality. (Wiring diagram: Main body (2/4): 23-O)

- Replace IDCS.

- Replacing the developing unit

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2824

Code

C-2824

Classification

Main body: Dot diameter correction abnormality

Cause

The output voltage of the IDC sensor (IDCS) is detected more than the specified value on the patch/K of the MPC maximum output.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing unit/K

- IDC sensor (IDCS)

- Color registration shutter solenoid (SD4)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check the installation of the developing unit and I/O, and repair or replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and IDCS, and repair it when there is any abnormality.

- Check the I/O of SD4 and repair or replace it when there is any abnormality.

- Check the sensor and clean or replace it when there is any dirt or abnormality. (Wiring diagram: Main body (2/4): 23-O)

- Replace IDCS.

- Replacing the developing unit

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2831

Code

C-2831

Classification

Main body: Dot diameter correction abnormality

Cause

The proper value cannot be figured out within the specified time by the correction of the dot diameter correction/Y.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing unit/Y

- IDC sensor (IDCS)

- Color registration shutter solenoid (SD4)

- Writing unit/Y

- Printer image processing board (PRIPB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check that the writing unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check the installation of the developing unit and I/O, and repair or replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and IDCS, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the writing unit and PRIPB, and repair it when there is any abnormality.

- Check the I/O of SD4 and repair or replace it when there is any abnormality.

- Check the sensor and clean or replace it when there is any dirt or abnormality. (Wiring diagram: Main body (2/4): 23-O)

- Replace IDCS.

- Replacing the developing unit

- Replace PRIPB.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2832

Code

C-2832

Classification

Main body: Dot diameter correction abnormality

Cause

The proper value cannot be figured out within the specified time by the correction of the dot diameter correction/M.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing unit/M

- IDC sensor (IDCS)

- Color registration shutter solenoid (SD4)

- Writing unit/M

- Printer image processing board (PRIPB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check that the writing unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check the installation of the developing unit and I/O, and repair or replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and IDCS, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the writing unit and PRIPB, and repair it when there is any abnormality.

- Check the I/O of SD4 and repair or replace it when there is any abnormality.

- Check the sensor and clean or replace it when there is any dirt or abnormality. (Wiring diagram: Main body (2/4): 23-O)

- Replace IDCS.

- Replacing the developing unit

- Replace PRIPB.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2833

Code

C-2833

Classification

Main body: Dot diameter correction abnormality

Cause

The proper value cannot be figured out within the specified time by the correction of the dot diameter correction/C.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing unit/C

- IDC sensor (IDCS)

- Color registration shutter solenoid (SD4)

- Writing unit/C

- Printer image processing board (PRIPB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check that the writing unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check the installation of the developing unit and I/O, and repair or replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and IDCS, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the writing unit and PRIPB, and repair it when there is any abnormality.

- Check the I/O of SD4 and repair or replace it when there is any abnormality.

- Check the sensor and clean or replace it when there is any dirt or abnormality. (Wiring diagram: Main body (2/4): 23-O)

- Replace IDCS.

- Replacing the developing unit

- Replace PRIPB.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2834

Code

C-2834

Classification

Main body: Dot diameter correction abnormality

Cause

The proper value cannot be figured out within the specified time by the correction of the dot diameter correction/K.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Developing unit/K

- IDC sensor (IDCS)

- Color registration shutter solenoid (SD4)

- Writing unit/K

- Printer image processing board (PRIPB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check that the writing unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check the installation of the developing unit and I/O, and repair or replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and IDCS, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the writing unit and PRIPB, and repair it when there is any abnormality.

- Check the I/O of SD4 and repair or replace it when there is any abnormality.

- Check the sensor and clean or replace it when there is any dirt or abnormality. (Wiring diagram: Main body (2/4): 23-O)

- Replace IDCS.

- Replacing the developing unit

- Replace PRIPB.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2840

Code

C-2840

Classification

Main body: Gamma correction abnormality

Cause

On the IDC sensor (IDCS) base correction, it is detected that the IDC sensor (IDCS) output value is less than the specified range after the rough adjustment.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- IDC sensor (IDCS)

- Color registration shutter solenoid (SD4)

- Intermediate Transfer Belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check that the belt cleaner blade on the intermediate transfer section is not curled and reinstall or repair it when there is any abnormality.

- Check the intermediate transfer belt, and clean or replace it if there is any dirt or abnormality.

- Check the connector connection and the wiring between PRCB and IDCS, and repair it when there is any abnormality.

- Check the I/O of SD4 and repair or replace it when there is any abnormality.

- Check the sensor and clean or replace it when there is any dirt or abnormality. (Wiring diagram: Main body (2/4): 23-O)

- Replace IDCS.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-2841

Code

C-2841

Classification

Main body: Gamma correction abnormality

Cause

On the IDC sensor (IDCS) base correction, it is detected that the IDC sensor (IDCS) output value is other than the specified range for specified times in a row. Alternatively, the LED applied voltage is out of the specified value for specified times in a row.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- IDC sensor (IDCS)

- Intermediate Transfer Belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the process unit is correctly installed without looseness, and reinstall or repair it when there is any abnormality.

- Check that the belt cleaner blade on the intermediate transfer section is not curled and reinstall or repair it when there is any abnormality.

- Check the intermediate transfer belt, and clean or replace it if there is any dirt or abnormality.

- Check the connector connection and the wiring between PRCB and IDCS, and repair it when there is any abnormality.

- Check the sensor and clean or replace it when there is any dirt or abnormality. (Wiring diagram: Main body (2/4): 23-O)

- Replace IDCS.

- Replace PRCB.

- Replace IDCS and PRCB at a time

Faulty part isolation DIPSW Control while detached

  - Solution 7 (C-3001_3500)

    - C-3101

Code

C-3101

Classification

Main body: Motor abnormality

Cause

When the fusing motor (M46) is active, an error detection signal of M46 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Fusing motor (M46)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the fusing unit, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the motor and the PRCB, and repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 1-H)

- Replace M46.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3102

Code

C-3102

Classification

Main body: Motor abnormality

Cause

During the pressing operation: Within a specified period of time after the fusing pressure release motor (M47) becomes active, the fusing pres- sure position sensor/2 (PS26) or the fusing pressure position sensor/3 (PS25) does not become active. During the release operation: Within a specified period of time after M47 becomes active, the fusing pressure position sensor/1 (PS24) does not become active.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Fusing pressure release motor (M47)

- Fusing pressure position sensor/1 (PS24)

- Fusing pressure position sensor/2 (PS26)

- Fusing pressure position sensor/3 (PS25)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the front door is closed properly, and repair it when there is any abnormality.

- Check the fusing unit, and repair it when there is any abnormality.

- Check the fusing drive gear system, and repair it when there is any abnormality.

- Check the connector connection and the wiring among the motor, the sensor, and the PRCB. Then repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 1-I)

- Check I/O and the operation of the sensor, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 1-G, (1/4): 1-J)

- Replace PS24, PS26, and PS25.

- Replace M47.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3103

Code

C-3103

Classification

Main body: Motor abnormality

Cause

When the 2nd transfer roller pressure release home sensor (PS19) is active: Within a specified period of time after the 2nd transfer pressure release motor (M38) becomes active, PS19 does not become inactive. When PS19 is inactive: Within a specified period of time after M38 becomes active, PS19 does not become active.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Conveyance drive board (CDB)

- 2nd transfer pressure release motor (M38)

- 2nd transfer roller pressure release home sensor (PS19)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the front door is closed properly, and repair it when there is any abnormality.

- Check the 2nd transfer intermediate transfer unit, and repair it when there is any abnormality.

- Check the connector connection and the wiring between CDB and the motor the sensor, and repair it when there is any abnormality.

- Check I/O of the sensor, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (1/4): 20-P)

- Check the I/O drive of the motor and the coupling of the gear, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (1/4): 20-P)

- Replace PS19.

- Replace M38.

- Replace CDB.

- Replace M38 and CDB at a time. Faulty part isolation DIPSW Control while detached

    - C-3104

Code

C-3104

Classification

Main body: Motor abnormality

Cause

When the fusing pressure release motor (M47) is active, 2 or more of the following sensors become active: the fusing pressure position sensor/1 (PS24), the fusing pressure position sensor/2 (PS26) and the fusing pressure position sensor/3 (PS25).

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Fusing pressure release motor (M47)

- Fusing pressure position sensor/1 (PS24)

- Fusing pressure position sensor/2 (PS26)

- Fusing pressure position sensor/3 (PS25)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the front door is closed properly, and repair it when there is any abnormality.

- Check the fusing unit, and repair it when there is any abnormality.

- Check the fusing drive gear system, and repair it when there is any abnormality.

- Check the connector connection and the wiring among the motor, the sensor, and the PRDB. Then repair it when there is any abnormality.

- Check the I/O drive of the motor and the coupling of the gear, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 1-I)

- Check I/O and the operation of the sensor, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 1-G, (1/4): 1-J)

- Replace PS24, PS26, or PS25.

- Replace M47.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3301*

Code

C-3301*

Classification

Main body: Fan abnormality

Cause

When the fusing exhaust fan (FM8) is active, an error detection signal of FM8 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Fusing exhaust fan (FM8)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between the fan and PRCB, and repair it when there is any abnormality.

- Rotate the fan or I/O manually, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 10-E)

- Replace FM8.

- Replace PRCB.

- Replace FM8 and PRCB at a time.

Faulty part isolation DIPSW Control while detached

    - C-3302*

Code

C-3302*

Classification

Main body: Fan abnormality

Cause

When the fusing belt exhaust fan (FM9) is active, an error detection signal of FM9 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Fusing belt ventilation fan (FM9)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between the fan and PRCB, and repair it when there is any abnormality.

- Rotate the fan or I/O manually, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 11-E)

- Replace FM9.

- Replace PRCB.

- Replace FM9 and PRCB at a time.

Faulty part isolation DIPSW Control while detached

    - C-3303

Code

C-3303

Classification

Main body: Fan abnormality

Cause

When the fusing roller cooling fan/1 (FM33) is active, an error detection signal of FM33 is detected continuously for a specified period of time.When the fusing roller cooling fan/2 (FM34) is active, an error detection signal of FM34 is detected continuously for a specified period of time. Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Conveyance drive board (CDB)

- Printer control board (PRCB)

- Fusing roller cooling fan/1 (FM33)

- Fusing roller cooling fan/2 (FM34)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring among the fan, CDB and PRCB, and repair it when there is any abnormality.

- Rotate the fan or I/O manually, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 2-K)

- Replace FM33 and FM34.

- Replace CDB.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3304*

Code

C-3304*

Classification

Main body: Fan abnormality

Cause

When the fusing separation fan/1 (FM10) is active, an error detection signal of FM10 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Fusing separation fan/1 (FM10)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between the fan and PRCB, and repair it when there is any abnormality.

- Rotate the fan or I/O manually, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 11-E)

- Replace FM10.

- Replace PRCB.

- Replace FM10 and PRCB at a time.

Faulty part isolation DIPSW Control while detached

    - C-3305*

Code

C-3305*

Classification

Main body: Fan abnormality

Cause

When the fusing separation fan/2 (FM11) is active, an error detection signal of FM11 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Fusing separation fan/2 (FM11)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between the fan and PRCB, and repair it when there is any abnormality.

- Rotate the fan by I/O or manually, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 12-E)

- Replace FM11.

- Replace PRCB.

- Replace FM11 and PRCB at a time.

Faulty part isolation DIPSW Control while detached

    - C-3307*

Code

C-3307*

Classification

Main body: Fan abnormality

Cause

When the fusing separation fan/3 (FM12) is active, an error detection signal of FM12 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Fusing separation fan/3 (FM12)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between the fan and PRCB, and repair it when there is any abnormality.

- Rotate the fan by I/O or manually, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 12-E)

- Replace FM12.

- Replace PRCB.

- Replace FM12 and PRCB at a time.

Faulty part isolation DIPSW Control while detached

    - C-3308*

Code

C-3308*

Classification

Main body: Fan abnormality

Cause

When the fusing intake fan (FM13) is active, an error detection signal of FM13 is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Fusing intake fan (FM13)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between the fan and PRCB, and repair it when there is any abnormality.

- Rotate the fan by I/O or manually, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 12-L)

- Replace FM13.

- Replace PRCB.

- Replace FM13 and PRCB at a time.

Faulty part isolation DIPSW Control while detached

    - C-3401

Code

C-3401

Classification

Main body: Fusing unit incorrect insertion abnormality

Cause

The fusing unit for C1070 series is inserted. Measures to take when an alert occurs The main body and the option stop instantly. Estimated abnormal parts

- Fusing unit

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

1. Check the fusing unit that is inserted, and correct it if it is one of C1070 series.

Faulty part isolation DIPSW Control while detached

  - Solution 8 (C-3501_4000)

    - C-3501 *

Code

C-3501*

Classification

Main body: Fusing high temperature abnormality

Cause

Fusing high temperature abnormality

The fusing temperature sensor/1 (TH1) detects that the temperature is 240 °C or higher continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/1 (FUSCB1) (Only for Europe)

- Fusing temperature sensor/1 (TH1)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4):10-B)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB1, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-C) (Only for Europe)

- Check the installation condition of the fusing heater lamp and ON or OFF by I/O, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 3-C, 3-D)

- Check the installation condition of TH1 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4):1-E)

- Replace TH1.

- Replace FUSCB1 (Only for Europe).

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3502 *

Code

C-3502*

Classification

Main body: Fusing high temperature abnormality

Cause

Fusing high temperature abnormality

The fusing temperature sensor/2 (TH2) detects that the temperature is 240 °C or higher continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/2 (FUSCB2) (Only for Europe)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing temperature sensor/2 (TH2)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4):10-B)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB2, the FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-C, 10-D) (Only for Europe)

- Check the installation condition of the fusing heater lamp and ON or OFF by I/O, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 3-C, 3-D)

- Check the installation condition of TH2 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4):1-E)

- Replace TH2.

- Replace FUSCB2 and FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3503*

Code

C-3503*

Classification

Main body: Fusing high temperature abnormality

Cause

Fusing high temperature abnormality

The fusing temperature sensor/5 (TH5) detects that the temperature is 200 °C or higher continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply /1 (DCPS1)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing temperature sensor/5 (TH5)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 7-G)

- Check the connector connection and the wiring among the fusing heater lamp, FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-C) (Only for Europe)

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 2-F)

- Check the installation condition of TH5 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-H)

- Replace TH5.

- Replace FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3504*

Code

C-3504*

Classification

Main body: Fusing high temperature abnormality

Cause

Fusing high temperature abnormality

The fusing temperature sensor/4 (TH4) detects that the temperature is 200 °C or higher continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply /1 (DCPS1)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing temperature sensor/4 (TH4)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 7-G)

- Check the connector connection and the wiring among the fusing heater lamp, FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-D) (Only for Europe)

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 2-F)

- Check the installation condition of TH4 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-G)

- Replace TH4.

- Replace FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3509*

Code

C-3509*

Classification

Main body: Fusing temperature difference abnormality

Cause

The temperature difference between the fusing temperature sensor/1 (TH1) and the fusing temperature sensor/3 (TH3) stays 100°C or more for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/1 (FUSCB1) (Only for Europe)

- Fusing control board/2 (FUSCB2) (Only for Europe)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing heater lamp/1 (L1)

- Fusing heater lamp/2 (L2)

- Fusing heater lamp/3 (L3)

- Fusing temperature sensor/1 (TH1)

- Fusing temperature sensor/3 (TH3)

- Thermostat/1 (TS1)

- Thermostat/2 (TS2)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the front door is not opened or closed frequently, and wait for several minutes while the front door is closed.

- Check the fusing roller, and repair it when there is any abnormality such as a paper wrap.

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 3-C, 3-D)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB1, the FUSCB2, the FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-C, 10-D) (Only for Europe)

- Check the conduction of TS1 and TS2, and repair it when there is any abnormality.

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 3-C, 3-D)

- Check the installation condition of TH1 and TH3, and the temperature of I/O. Then adjust it when there is any abnormality. (Wiring diagram: Main body (1/4): 1-D, 1-E)

- Replace TH1 and TH3.

- Replace TS1 and TS2.

- Replace the fusing heater lamp assy/Up.

- Replace FUSCB1, FUSCB2, and FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

- Replace the fusing unit. Faulty part isolation DIPSW Control while detached

    - C-3510*

Code

C-3510*

Classification

Main body: Fusing temperature difference abnormality

Cause

The temperature difference between the fusing temperature sensor/2 (TH2) and the fusing temperature sensor/3 (TH3) stays 100°C or more for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/2 (FUSCB2) (Only for Europe)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing heater lamp/2 (L2)

- Fusing heater lamp/3 (L3)

- Fusing temperature sensor/2 (TH2)

- Fusing temperature sensor/3 (TH3)

- Thermostat/2 (TS2)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the front door is not opened or closed frequently, and wait for several minutes while the front door is closed.

- Check the fusing roller, and repair it when there is any abnormality such as a paper wrap.

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 3-C, 3-D)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB2, the FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-C, 10-D) (Only for Europe)

- Check the conduction of TS2, and repair it when there is any abnormality.

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 3-C, 3-D)

- Check the installation condition of TH2 and TH3, and the temperature of I/O. Then adjust it when there is any abnormality. (Wiring diagram: Main body (1/4): 1-D, 1-E)

- Replace TH2 and TH3.

- Replace TS2.

- Replace the fusing heater lamp assy/Up.

- Replace FUSCB2 and FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

- Replace the fusing unit. Faulty part isolation DIPSW Control while detached

    - C-3511*

Code

C-3511*

Classification

Main body: Fusing temperature difference abnormality

Cause

The temperature difference between the fusing temperature sensor/1 (TH1) and the fusing temperature sensor/2 (TH2) stays 100°C or more for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/1 (FUSCB1) (Only for Europe)

- Fusing control board/2 (FUSCB2) (Only for Europe)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing heater lamp/1 (L1)

- Fusing heater lamp/2 (L2)

- Fusing heater lamp/3 (L3)

- Fusing temperature sensor/1 (TH1)

- Fusing temperature sensor/2 (TH2)

- Thermostat/1 (TS1)

- Thermostat/2 (TS2)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the front door is not opened or closed frequently, and wait for several minutes while the front door is closed.

- Check the fusing roller, and repair it when there is any abnormality such as a paper wrap.

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 3-C, 3-D)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB1, the FUSCB2, the FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-C, 10-D) (Only for Europe)

- Check the conduction of TS1 and TS2, and repair it when there is any abnormality.

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 3-C, 3-D)

- Check the installation condition of TH1 and TH2, and the temperature of I/O. Then adjust it when there is any abnormality. (Wiring diagram: Main body (1/4): 1-E)

- Replace TH1 and TH2.

- Replace TS1 and TS2.

- Replace the fusing heater lamp assy/Up.

- Replace FUSCB1, FUSCB2, and FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

- Replace the fusing unit. Faulty part isolation DIPSW Control while detached

    - C-3512*

Code

C-3512*

Classification

Main body: Fusing high temperature abnormality

Cause

Fusing high temperature abnormality.

The fusing temperature sensor/3 (TH3) detects that the temperature is 240 °C or higher continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/2 (FUSCB2) (Only for Europe)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing temperature sensor/3 (TH3)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 10-B)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB2, the FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-C, 10-D) (Only for Europe)

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 3-C, 3-D)

- Check the installation condition of TH3 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-D)

- Replace TH3.

- Replace FUSCB2 and FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3513*

Code

C-3513*

Classification

Main body: Fusing wiring disconnection abnormality

Cause

Within a specified period of time after the warm-up starts, the AD value of the fusing temperature sensor/1 (TH1) is changed abnormally and the wiring disconnection is detected.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/1 (FUSCB1) (Only for Europe)

- Fusing heater lamp/1 (L1)

- Fusing temperature sensor/1 (TH1)

- Thermostat/1 (TS1)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 10-B)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB1, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-C) (Only for Europe)

- Check the conduction of TS1, and repair it when there is any abnormality.

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 3-C, 3-D)

- Check the installation condition of TH1 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-E)

- Replace TH1.

- Replace TS1.

- Replace the fusing heater lamp assy/Up.

- Replace FUSCB1. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3515*

Code

C-3515*

Classification

Main body: Fusing wiring disconnection abnormality

Cause

Within a specified period of time after the warm-up starts, the AD value of the fusing temperature sensor/2 (TH2) is changed abnormally and the wiring disconnection is detected.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/2 (FUSCB2) (Only for Europe)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing heater lamp/2 (L2)

- Fusing heater lamp/3 (L3)

- Fusing temperature sensor/2 (TH2)

- Thermostat/2 (TS2)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 10-B)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB2, the FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-C, 10-D) (Only for Europe)

- Check the conduction of TS2, and repair it when there is any abnormality.

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 3-C, 3-D)

- Check the installation condition of TH2 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-E)

- Replace TH2.

- Replace TS2.

- Replace the fusing heater lamp assy/Up.

- Replace FUSCB2 and FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3517*

Code

C-3517*

Classification

Main body: Fusing wiring disconnection abnormality

Cause

Within a specified period of time after the warm-up starts, the AD value of the fusing temperature sensor/3 (TH3) is changed abnormally and the wiring disconnection is detected.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/2 (FUSCB2) (Only for Europe)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing heater lamp/2 (L2)

- Fusing heater lamp/3 (L3)

- Fusing temperature sensor/3 (TH3)

- Thermostat/2 (TS2)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 10-B)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB2, the FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-C, 10-D) (Only for Europe)

- Check the conduction of TS2, and repair it when there is any abnormality.

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 3-C, 3-D)

- Check the installation condition of TH3 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-D)

- Replace TH3.

- Replace TS2.

- Replace the fusing heater lamp assy/Up.

- Replace FUSCB2 and FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3518*

Code

C-3518*

Classification

Main body: Fusing wiring disconnection abnormality

Cause

Within a specified period of time after the warm-up starts, the AD value of the fusing temperature sensor/5 (TH5) is changed abnormally and the wiring disconnection is detected.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing heater lamp/4 (L4)

- Fusing temperature sensor/5 (TH5)

- Thermostat/3 (TS3)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 10-B)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-D) (Only for Europe)

- Check the conduction of TS3, and repair it when there is any abnormality.

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 2-F)

- Check the installation condition of TH5 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-H)

- Replace TH5.

- Replace TS3.

- Replace L4.

- Replace FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3519*

Code

C-3519*

Classification

Main body: Fusing wiring disconnection abnormality

Cause

Within a specified period of time after the warm-up starts, the AD value of the fusing temperature sensor/4 (TH4) is changed abnormally and the wiring disconnection is detected.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing heater lamp/4 (L4)

- Fusing temperature sensor/4 (TH4)

- Thermostat/3 (TS3)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 10-B)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-D) (Only for Europe)

- Check the conduction of TS3, and repair it when there is any abnormality.

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 2-F)

- Check the installation condition of TH4 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-G)

- Replace TH4.

- Replace TS3.

- Replace L4.

- Replace FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3801*

Code

C-3801*

Classification

Main body: Fusing low temperature abnormality

Cause

Fusing low temperature abnormality.

During the fusing idling or printing, the fusing temperature sensor/1 (TH1) detects that the temperature is less than 100°C continuously for a spec- ified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/1 (FUSCB1) (Only for Europe)

- Fusing heater lamp/1 (L1)

- Fusing temperature sensor/1 (TH1)

- Thermostat/1 (TS1)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 3-C, 3-D)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB1, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-C) (Only for Europe)

- Check the conduction of TS1, and repair it when there is any abnormality.

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 3-C, 3-D)

- Check the installation condition of TH1 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-E)

- Replace TH1.

- Replace TS1.

- Replace the fusing heater lamp assy/Up.

- Replace FUSCB1. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3802*

Code

C-3802*

Classification

Main body: Fusing low temperature abnormality

Cause

Fusing low temperature abnormality.

During the fusing idling or printing, the fusing temperature sensor/2 (TH2) detects that the temperature is less than 50°C continuously for a spec- ified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/2 (FUSCB2) (Only for Europe)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing heater lamp/2 (L2)

- Fusing heater lamp/3 (L3)

- Fusing temperature sensor/2 (TH2)

- Thermostat/2 (TS2)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 3-C, 3-D)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB2, the DCPS1, and the FUSCB3. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-C, 10-D) (Only for Europe)

- Check the conduction of TS2, and repair it when there is any abnormality.

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 3-C, 3-D)

- Check the installation condition of TH2 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-E)

- Replace TH2.

- Replace TS2.

- Replace the fusing heater lamp assy/Up.

- Replace FUSCB2 and FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3803*

Code

C-3803*

Classification

Main body: Fusing low temperature abnormality

Cause

Fusing low temperature abnormality.

During the fusing idling or printing, the fusing temperature sensor/5 (TH5) detects that the temperature is less than 50°C continuously for a spec- ified period of time. However, the detection is not performed in the monochrome energy-save mode.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing heater lamp/4 (L4)

- Fusing temperature sensor/5 (TH5)

- Thermostat/3 (TS3)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 2-F)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-D) (Only for Europe)

- Check the conduction of TS3, and repair it when there is any abnormality.

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 2-F)

- Check the installation condition of TH5 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4):1-H)

- Replace TH5.

- Replace TS3.

- Replace L4.

- Replace FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3807*

Code

C-3807*

Classification

Main body: Fusing low temperature abnormality

Cause

Fusing low temperature abnormality.

During the fusing idling or printing, the fusing temperature sensor/3 (TH3) detects that the temperature is less than 50°C continuously for a spec- ified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/2 (FUSCB2) (Only for Europe)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing heater lamp/2 (L2)

- Fusing heater lamp/3 (L3)

- Fusing temperature sensor/3 (TH3)

- Thermostat/2 (TS2)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 3-C, 3-D)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB2, the DCPS1, and the FUSCB3. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-C, 10-D) (Only for Europe)

- Check the conduction of TS2, and repair it when there is any abnormality.

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 3-C, 3-D)

- Check the installation condition of TH3 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-D)

- Replace TH3.

- Replace TS2.

- Replace the fusing heater lamp assy/Up.

- Replace FUSCB2 and FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3901*

Code

C-3901*

Classification

Main body: Fusing high temperature abnormality (Hardware detection)

Cause

The fusing temperature sensor/1 (TH1) detects an error signal continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/1 (FUSCB1) (Only for Europe)

- Fusing temperature sensor/1 (TH1)

Correction  WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 3-C, 3-D)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB1, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-C) (Only for Europe)

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 3-C, 3-D)

- Check the installation condition of TH1 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-E)

- Replace TH1.

- Replace FUSCB1 (Only for Europe).

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3902*

Code

C-3902*

Classification

Main body: Fusing low temperature abnormality (Hardware detection)

Cause

The fusing temperature sensor/1 (TH1) detects an error signal continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/1 (FUSCB1) (Only for Europe)

- Fusing heater lamp/1 (L1)

- Fusing temperature sensor/1 (TH1)

- Thermostat/1 (TS1)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 3-C, 3-D)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB1, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-C) (Only for Europe)

- Check the conduction of TS1, and repair it when there is any abnormality.

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 3-C, 3-D)

- Check the installation condition of TH1 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-E)

- Replace TH1.

- Replace TS1.

- Replace the fusing heater lamp assy/Up.

- Replace FUSCB1 (Only for Europe).

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3903*

Code

C-3903*

Classification

Main body: Fusing high temperature abnormality (Hardware detection)

Cause

The fusing temperature sensor/2 (TH2) detects an error detection signal continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/2 (FUSCB2) (Only for Europe)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing temperature sensor/2 (TH2)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 3-C, 3-D)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB2, the FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-C, 10-D) (Only for Europe)

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 3-C, 3-D)

- Check the installation condition of TH2 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-E)

- Replace TH2.

- Replace FUSCB2 and FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3904*

Code

C-3904*

Classification

Main body: Fusing low temperature abnormality (Hardware detection)

Cause

The fusing temperature sensor/2 (TH2) detects an error detection signal continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/2 (FUSCB2) (Only for Europe)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing heater lamp/2 (L2)

- Fusing heater lamp/3 (L3)

- Fusing temperature sensor/2 (TH2)

- Thermostat/2 (TS2)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 3-C, 3-D)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB2, the FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-C) (Only for Europe)

- Check the conduction of TS2, and repair it when there is any abnormality.

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 3-C, 3-D)

- Check the installation condition of TH2 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-E)

- Replace TH2.

- Replace TS2.

- Replace the fusing heater lamp assy/Up.

- Replace FUSCB2 and FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3905*

Code

C-3905*

Classification

Main body: Fusing high temperature abnormality (Hardware detection)

Cause

The fusing temperature sensor/3 (TH3) detects an error signal continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/2 (FUSCB2) (Only for Europe)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing temperature sensor/3 (TH3)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 3-C, 3-D)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB2, the FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-C, 10-D) (Only for Europe)

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 3-C, 3-D)

- Check the installation condition of TH3 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-D)

- Replace TH3.

- Replace FUSCB2 and FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3906*

Code

C-3906*

Classification

Main body: Fusing low temperature abnormality (Hardware detection)

Cause

The fusing temperature sensor/3 (TH3) detects an error signal continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/2 (FUSCB2) (Only for Europe)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing heater lamp/2 (L2)

- Fusing heater lamp/3 (L3)

- Fusing temperature sensor/3 (TH3)

- Thermostat/2 (TS2)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 3-C, 3-D)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB2, the FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-C) (Only for Europe)

- Check the conduction of TS2, and repair it when there is any abnormality.

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 3-C, 3-D)

- Check the installation condition of TH3 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-D)

- Replace TH3.

- Replace TS2.

- Replace the fusing heater lamp assy/Up.

- Replace FUSCB2 and FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3907*

Code

C-3907*

Classification

Main body: Fusing high temperature abnormality (Hardware detection)

Cause

The fusing temperature sensor/5 (TH5) detects an error detection signal continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing temperature sensor/5 (TH5)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 2-F)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-D) (Only for Europe)

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 2-F)

- Check the installation condition of TH5 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-H)

- Replace TH5.

- Replace FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3908*

Code

C-3908*

Classification

Main body: Fusing low temperature abnormality (Hardware detection)

Cause

The fusing temperature sensor/5 (TH5) detects an error detection signal continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing heater lamp/4 (L4)

- Fusing temperature sensor/5 (TH5)

- Thermostat/3 (TS3)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 2-F)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-D) (Only for Europe)

- Check the conduction of TS3, and repair it when there is any abnormality.

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 2-F)

- Check the installation condition of TH5 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-H)

- Replace TH5.

- Replace TS3.

- Replace L4.

- Replace FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3909*

Code

C-3909*

Classification

Main body: Fusing high temperature abnormality (Hardware detection)

Cause

The fusing temperature sensor/4 (TH4) detects an error signal continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing temperature sensor/4 (TH4)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 2-F)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-D) (Only for Europe)

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 2-F)

- Check the installation condition of TH4 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-G)

- Replace TH4.

- Replace FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3910*

Code

C-3910*

Classification

Main body: Fusing low temperature abnormality (Hardware detection)

Cause

The fusing temperature sensor/4 (TH4) detects an error signal continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing heater lamp/4 (L4)

- Fusing temperature sensor/4 (TH4)

- Thermostat/3 (TS3)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 2-F)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-D) (Only for Europe)

- Check the conduction of TS3, and repair it when there is any abnormality.

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 2-F)

- Check the installation condition of TH4 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-G)

- Replace TH4.

- Replace TS3.

- Replace L4.

- Replace FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3911*

Code

C-3911*

Classification

Main body: The fusing warm up abnormality

Cause

Within a specified period of time after the warm-up starts, the temperature that the fusing temperature sensor/1 (TH1) detects does not reach the prescribed value.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/1 (FUSCB1) (Only for Europe)

- Fusing heater lamp/1 (L1)

- Fusing temperature sensor/1 (TH1)

- Thermostat/1 (TS1)

Correction  WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 3-C, 3-D)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB1, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-C) (Only for Europe)

- Check the conduction of TS1, and repair it when there is any abnormality.

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 3-C, 3-D)

- Check the installation condition of TH1 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-E)

- Replace TH1.

- Replace TS1.

- Replace the fusing heater lamp assy/Up.

- Replace DCPS1.

- Replace FUSCB1. (Only for Europe)

- Replace PRCB.

- Replace the fusing heater lamp assy/Up and DCPS1 at a time.

Faulty part isolation DIPSW Control while detached

    - C-3912*

Code

C-3912*

Classification

Main body: The fusing warm up abnormality

Cause

Within a specified period of time after the warm-up starts, the temperature that the fusing temperature sensor/4 (TH4) detects does not reach 40

°C. However, the detection is not performed in the monochrome energy-save mode.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing heater lamp/4 (L4)

- Fusing temperature sensor/4 (TH4)

- Thermostat/3 (TS3)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1, and repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 2-F)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-D) (Only for Europe)

- Check the conduction of TS3, and repair it when there is any abnormality.

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 2-F)

- Check the installation condition of TH4 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-G)

- Replace TH4.

- Replace TS3.

- Replace L4.

- Replace FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-3913*

Code

C-3913*

Classification

Main body: Fusing unit placement abnormality

Cause

The fusing unit is not installed.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the fusing unit is properly installed, and repair it when there is any abnormality.

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring on PRCB, and repair it when there is any abnormality.

- Replace PRCB.

- Replace the fusing unit. Faulty part isolation DIPSW Control while detached

    - C-3917*

Code

C-3917*

Classification

Main body: The fusing warm up abnormality

Cause

Within a specified period of time after the warm-up starts, the temperature that the fusing temperature sensor/2 (TH2) detects does not reach the prescribed value.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Fusing unit

- Printer control board (PRCB)

- DC power supply/1 (DCPS1)

- Fusing control board/2 (FUSCB2) (Only for Europe)

- Fusing control board/3 (FUSCB3) (Only for Europe)

- Fusing heater lamp/2 (L2)

- Fusing heater lamp/3 (L3)

- Fusing temperature sensor/2 (TH2)

- Thermostat/2 (TS2)

Correction

WARNING

- Be sure to select the DIPSW3-1 to 0 after you replace the defective parts. If DIPSW3-1 is selected to 0 before defective parts are fixed, it possibly cause a fire.

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connection between the rear side connector of the fusing unit and the main body side connector, and repair it when there is any abnormality.

- Check the connector connection and the wiring between the fusing temperature sensor and the PRCB, and repair it when there is any abnor- mality.

- Check the connector connection and the wiring between the fusing heater lamp and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (1/4): 3-C, 3-D)

- Check the connector connection and the wiring among the fusing heater lamp, the FUSCB2, the FUSCB3, and the DCPS1. Then repair it when there is any abnormality. (Wiring diagram: Main body (4/4): 10-A, 10-C, 10-D) (Only for Europe)

- Check the conduction of TS2, and repair it when there is any abnormality.

- Check the installation condition of the fusing heater lamp and ON or OFF by the I/O, and repair it when there is any abnormality. (Wiring dia- gram: Main body (1/4): 3-C, 3-D)

- Check the installation condition of TH2 and the temperature of I/O, and adjust it when there is any abnormality. (Wiring diagram: Main body (1/ 4): 1-E)

- Replace TH2.

- Replace TS2.

- Replace the fusing heater lamp assy/Up.

- Replace FUSCB2 and FUSCB3. (Only for Europe)

- Replace DCPS1.

- Replace PRCB.

- Replace the fusing heater lamp assy/Up and DCPS1 at a time.

Faulty part isolation DIPSW Control while detached

  - Solution 9 (C-4001_5000)

    - C-4101

Code

C-4101

Classification

Main body: Polygon motor abnormality

Cause

When the polygon motor/Y (M20) in the writing unit is booted, the lock signal of M20 is not detected within a specified period of time. Alternatively, after the boot process, it is detected continuously for a specified period of time that a lock signal of M20 becomes inactive.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Polygon motor/Y (M20)

- Printer control board (PRCB)

- DC power supply/2 (DCPS/2)

- Writing unit/Y

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the writing unit, and replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the writing unit, and repair it when there is any abnormality.

- Check the connector connection and the wiring between DCPS/2 and the writing unit, and repair it when there is any abnormality.

- Check the I/O drive of the motor, and replace the writing unit when there is any abnormality.

- Replace the writing unit.

- Replace PRCB.

- Replace DCPS/2.

Faulty part isolation DIPSW Control while detached

    - C-4102

Code

C-4102

Classification

Main body: Polygon motor abnormality

Cause

When the polygon motor/M (M21) in the writing unit is booted, the lock signal of M21 is not detected within a specified period of time. Alternatively, after the boot process, it is detected continuously for a specified period of time that a lock signal of M21 becomes inactive.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Polygon motor/M (M21)

- Printer control board (PRCB)

- DC power supply/2 (DCPS/2)

- Writing unit/M

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the writing unit, and replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the writing unit, and repair it when there is any abnormality.

- Check the connector connection and the wiring between DCPS/2 and the writing unit, and repair it when there is any abnormality.

- Check the I/O drive of the motor, and replace the writing unit when there is any abnormality.

- Replace the writing unit.

- Replace PRCB.

- Replace DCPS/2.

Faulty part isolation DIPSW Control while detached

    - C-4103

Code

C-4103

Classification

Main body: Polygon motor abnormality

Cause

When the polygon motor/C (M22) in the writing unit is booted, the lock signal of M22 is not detected within a specified period of time. Alternatively, after the boot process, it is detected continuously for a specified period of time that a lock signal of M22 becomes inactive.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Polygon motor/C (M22)

- Printer control board (PRCB)

- DC power supply/2 (DCPS/2)

- Writing unit/C

Correction Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the writing unit, and replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the writing unit, and repair it when there is any abnormality.

- Check the connector connection and the wiring between DCPS/2 and the writing unit, and repair it when there is any abnormality.

- Check the I/O drive of the motor, and replace the writing unit when there is any abnormality.

- Replace the writing unit.

- Replace PRCB.

- Replace DCPS/2.

Faulty part isolation DIPSW Control while detached

    - C-4104

Code

C-4104

Classification

Main body: Polygon motor abnormality

Cause

When the polygon motor/K (M23) in the writing unit is booted, the lock signal of M23 is not detected within a specified period of time. Alternatively, after the boot process, it is detected continuously for a specified period of time that a lock signal of M23 becomes inactive.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Polygon motor/K (M23)

- Printer control board (PRCB)

- DC power supply/2 (DCPS/2)

- Polygon motor/K

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the writing unit, and replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the writing unit, and repair it when there is any abnormality.

- Check the connector connection and the wiring between DCPS/2 and the writing unit, and repair it when there is any abnormality.

- Check the I/O drive of the motor, and replace the writing unit when there is any abnormality.

- Replace the writing unit.

- Replace PRCB.

- Replace DCPS/2.

Faulty part isolation DIPSW Control while detached

    - C-4501

Code

C-4501

Classification

Main body: Writing correction drive abnormality

Cause

When the tilt correction motor/Y (M24) in the writing unit is active, the tilt correction home sensor/Y (PS63) becomes inactive.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Writing unit

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the write unit, and replace it if there is any abnormality.

- Check the connector connection and the wiring between PRCB and the write unit, and repair it when there is any abnormality.

- Replace the write unit.

- Replace PRCB.

- Replace the writing unit and PRCB at a time.

Faulty part isolation DIPSW Control while detached

    - C-4502

Code

C-4502

Classification

Main body: Writing correction drive abnormality

Cause

When the tilt correction motor/M (M25) in the writing unit is active, the tilt correction home sensor/M (PS64) becomes inactive.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Writing unit

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the write unit, and replace it if there is any abnormality.

- Check the connector connection and the wiring between PRCB and the write unit, and repair it when there is any abnormality.

- Replace the write unit.

- Replace PRCB.

- Replace the writing unit and PRCB at a time.

Faulty part isolation DIPSW Control while detached

    - C-4503

Code

C-4503

Classification

Main body: Writing correction drive abnormality

Cause

When the tilt correction motor/C (M26) in the writing unit is active, the tilt correction home sensor/C (PS65) becomes inactive.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Writing unit

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the write unit, and replace it if there is any abnormality.

- Check the connector connection and the wiring between PRCB and the write unit, and repair it when there is any abnormality.

- Replace the write unit.

- Replace PRCB.

- Replace the writing unit and PRCB at a time.

Faulty part isolation DIPSW Control while detached

    - C-4504

Code

C-4504

Classification

Main body: Writing correction drive abnormality

Cause

When the tilt correction motor/K (M27) in the writing unit is active, the tilt correction home sensor/K (PS66) becomes inactive.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Writing unit

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the write unit, and replace it if there is any abnormality.

- Check the connector connection and the wiring between PRCB and the write unit, and repair it when there is any abnormality.

- Replace the write unit.

- Replace PRCB.

- Replace the writing unit and PRCB at a time.

Faulty part isolation DIPSW Control while detached

    - C-4505

Code

C-4505

Classification

Main body: Writing correction and drive correction range exceedance

Cause

The correction range exceedance is detected during the color registration correction (Y).

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Writing unit

- Intermediate transfer unit

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Note down the value of Writing Unit Skew Pos. (Y) from Service Mode [01 Machine Adjustment] - [01 Printer Adjustment] - [09 Writing Initial Pos. Memory] - [01 Skew Initial Position Memory].

Note

  - Do not press the start button.

  - Press [01 Skew Initial Position Memory] button after the initialization is completed (Approximately 10 seconds).

- Select I/O Check Mode to execute 35-17, 35-18, 35-19 in the order (move to the Initial Position).

Note

  - Do not execute 35-20.

- Execute [01 Machine Adjustment] - [01 Printer Adjustment] - [10 Color Registration Auto.] from Service Mode.

- When an error occurs, go to the solution procedure 7. When it ends normally, go to the next procedure.

- Execute the solution procedure 1 and note down the value again.

- When the difference of the two values is more than 50, end the operation. When the difference is 49 or less, go to the next procedure.

- Check the installation condition of the writing unit and the intermediate transfer unit, and repair it when there is any abnormality.

- Check the writing unit, and replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the writing unit, and repair it when there is any abnormality.

- Replace the writing unit.

- Replace PRCB.

- Replace writing unit and PRCB at a time

- Replace the intermediate transfer belt

Faulty part isolation DIPSW Control while detached

    - C-4506

Code

C-4506

Classification

Main body: Writing correction and drive correction range exceedance

Cause

The correction range exceedance is detected during the color registration correction (M).

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Writing unit

- Intermediate transfer unit

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Note down the value of Writing Unit Skew Pos. (M) from Service Mode [01 Machine Adjustment] - [01 Printer Adjustment] - [09 Writing Initial Pos. Memory] - [01 Skew Initial Position Memory].

Note

  - Do not press the start button.

  - Press [01 Skew Initial Position Memory] button after the initialization is completed (Approximately 10 seconds).

- Select I/O Check Mode to execute 35-17, 35-18, 35-19 in the order (move to the Initial Position).

Note

  - Do not execute 35-20.

- Execute [01 Machine Adjustment] - [01 Printer Adjustment] - [10 Color Registration Auto.] from Service Mode.

- When an error occurs, go to the solution procedure 7. When it ends normally, go to the next procedure.

- Execute the solution procedure 1 and note down the value again.

- When the difference of the two values is more than 50, end the operation. When the difference is 49 or less, go to the next procedure.

- Check the installation condition of the writing unit and the intermediate transfer unit, and repair it when there is any abnormality.

- Check the writing unit, and replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the writing unit, and repair it when there is any abnormality.

- Replace the writing unit.

- Replace PRCB.

- Replace writing unit and PRCB at a time

- Replace the intermediate transfer belt

Faulty part isolation DIPSW Control while detached

    - C-4507

Code

C-4507

Classification

Main body: Writing correction and drive correction range exceedance

Cause

The correction range exceedance is detected during the color registration correction (C).

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Writing unit

- Intermediate transfer unit

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Note down the value of Writing Unit Skew Pos. (C) from Service Mode [01 Machine Adjustment] - [01 Printer Adjustment] - [09 Writing Initial Pos. Memory] - [01 Skew Initial Position Memory].

Note

  - Do not press the start button.

  - Press [01 Skew Initial Position Memory] button after the initialization is completed (Approximately 10 seconds).

- Select I/O Check Mode to execute 35-17, 35-18, 35-19 in the order (move to the Initial Position).

Note

  - Do not execute 35-20.

- Execute [01 Machine Adjustment] - [01 Printer Adjustment] - [10 Color Registration Auto.] from Service Mode.

- When an error occurs, go to the solution procedure 7. When it ends normally, go to the next procedure.

- Execute the solution procedure 1 and note down the value again.

- When the difference of the two values is more than 50, end the operation. When the difference is 49 or less, go to the next procedure.

- Check the installation condition of the writing unit and the intermediate transfer unit, and repair it when there is any abnormality.

- Check the writing unit, and replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the writing unit, and repair it when there is any abnormality.

- Replace the writing unit.

- Replace PRCB.

- Replace writing unit and PRCB at a time

- Replace the intermediate transfer belt

Faulty part isolation DIPSW Control while detached

    - C-4508

Code

C-4508

Classification

Main body: Writing correction and drive correction range exceedance

Cause

The correction range exceedance is detected during the color registration correction (K).

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Writing unit

- Intermediate transfer unit

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the installation condition of the writing unit and the intermediate transfer unit, and repair it when there is any abnormality.

- Check the writing unit, and replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the writing unit, and repair it when there is any abnormality.

- Replace the writing unit.

- Replace PRCB.

- Replace writing unit and PRCB at a time

- Replace the intermediate transfer belt

Faulty part isolation DIPSW Control while detached

    - C-4511

Code

C-4511

Classification

Main body: Writing correction drive abnormality

Cause

Within a specified period of time after the activation, the tilt correction motor/Y (M24) does not become inactive. Alternatively, the installation posi- tion of the tilt correction motor/Y (PS63) is incorrect.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Tilt correction motor/Y (M24)

- Tilt correction home sensor/Y (PS63)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the write unit, and replace it if there is any abnormality.

- Check the connector connection and the wiring between PRCB and the write unit, and repair it when there is any abnormality.

- Check the I/O drive of the motor, and replace the write unit when there is any abnormality.

- Check the sensor, and replace the write unit when there is any abnormality.

- Replace the write unit.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-4512

Code

C-4512

Classification

Main body: Writing correction drive abnormality

Cause

Within a specified period of time after the activation, the tilt correction motor/M (M25) does not become inactive. Alternatively, the installation posi- tion of the tilt correction motor/M (PS64) is incorrect.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Tilt correction motor/M (M25)

- Tilt correction home sensor/M (PS64)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the write unit, and replace it if there is any abnormality.

- Check the connector connection and the wiring between PRCB and the write unit, and repair it when there is any abnormality.

- Check the I/O drive of the motor, and replace the write unit when there is any abnormality.

- Check the sensor, and replace the write unit when there is any abnormality.

- Replace the write unit.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-4513

Code

C-4513

Classification

Main body: Writing correction drive abnormality

Cause

Within a specified period of time after the activation, the tilt correction motor/C (M26) is not deactivated. Alternatively, the installation position of the tilt correction home sensor/C (PS65) is incorrect.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Tilt correction motor/C (M26)

- Tilt correction home sensor/C (PS65)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the write unit, and replace it if there is any abnormality.

- Check the connector connection and the wiring between PRCB and the write unit, and repair it when there is any abnormality.

- Check the I/O drive of the motor, and replace the write unit when there is any abnormality.

- Check the sensor, and replace the write unit when there is any abnormality.

- Replace the write unit.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-4514

Code

C-4514

Classification

Main body: Writing correction drive abnormality

Cause

Within a specified period of time after the activation, the tilt correction motor/K (M27) does not become inactive. Alternatively, the installation posi- tion of the tilt correction home sensor/K (PS66) is incorrect.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Tilt correction motor/K (M27)

- Tilt correction home sensor/K (PS66)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the write unit, and replace it if there is any abnormality.

- Check the connector connection and the wiring between PRCB and the write unit, and repair it when there is any abnormality.

- Check the I/O drive of the motor, and replace the write unit when there is any abnormality.

- Check the sensor, and replace the write unit when there is any abnormality.

- Replace the write unit.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-4515

Code

C-4515

Classification

Main body: Writing correction drive initial position abnormality

Cause

Within a specified period of time after the activation, the tilt correction motor/Y (M24) does not become inactive due to the edge detection during the initial position memory operation.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Tilt correction motor/Y (M24)

- Tilt correction home sensor/Y (PS63)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the writing unit, and replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the writing unit, and repair it when there is any abnormality.

- Check the I/O drive of the motor, and replace the writing unit when there is any abnormality.

- Check sensor, and replace the writing unit when there is any abnormality.

- Replace the writing unit.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-4516

Code

C-4516

Classification

Main body: Writing correction drive initial position abnormality

Cause

Within a specified period of time after the activation, the tilt correction motor/M (M25) is not deactivated due to the edge detection during the initial position memory operation.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Tilt correction motor/M (M25)

- Tilt correction home sensor/M (PS64)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the writing unit, and replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the writing unit, and repair it when there is any abnormality.

- Check the I/O drive of the motor, and replace the writing unit when there is any abnormality.

- Check sensor, and replace the writing unit when there is any abnormality.

- Replace the writing unit.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-4517

Code

C-4517

Classification

Main body: Writing correction drive initial position abnormality

Cause

Within a specified period of time after the activation, the tilt correction motor/C (M26) does not become inactive due to the edge detection during the initial position memory operation.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Tilt correction motor/C (M26)

- Tilt correction home sensor/C (PS65)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the writing unit, and replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the writing unit, and repair it when there is any abnormality.

- Check the I/O drive of the motor, and replace the writing unit when there is any abnormality.

- Check the sensor, and replace the writing unit when there is any abnormality.

- Replace the writing unit.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-4518

Code

C-4518

Classification

Main body: Writing correction drive initial position abnormality

Cause

Within a specified period of time after the activation, the tilt correction motor/K (M27) does not become inactive due to the edge detection during the initial position memory operation.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Tilt correction motor/K (M27)

- Tilt correction home sensor/K (PS66)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the writing unit, and replace it when there is any abnormality.

- Check the connector connection and the wiring between PRCB and the writing unit, and repair it when there is any abnormality.

- Check the I/O drive of the motor, and replace the writing unit when there is any abnormality.

- Check the sensor, and replace the writing unit when there is any abnormality.

- Replace the writing unit.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-4520

Code

C-4520

Classification

Main body: Color registration abnormality

Cause

The color registration correction has terminated abnormally.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Color registration sensor/Fr (PS60)

- Color registration sensor/Rr (PS62)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- When a SC occurs, store the log by the Log Store in the service mode.

- Report it with that log to KMBT.

Faulty part isolation DIPSW

Control while detached

    - C-4521

Code

C-4521

Classification

Main body: Color registration abnormality

Cause

The color registration base line correction/Fr has terminated abnormally.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Color registration sensor/Fr (PS60)

- Color registration shutter solenoid (SD4)

- Intermediate Transfer Belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the shutter and around the color registration sensor, and clean or replace it if there is any dirt or abnormality.

- Check the intermediate transfer belt, and clean or replace it if there is any dirt or abnormality.

- Check the connector connection and the wiring between PRCB and the sensor, and repair it when there is any abnormality.

- Check the I/O rate of the sensor by I/O of SD4, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 24-O)

- Replace PS60.

- Replace PRCB.

- Replace SD4 and PRCB at a time.

Faulty part isolation DIPSW Control while detached

    - C-4522

Code

C-4522

Classification

Main body: Color registration abnormality

Cause

The color registration base line correction/Rr has terminated abnormally.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Color registration sensor/Rr (PS62)

- Color registration shutter solenoid (SD4)

- Intermediate Transfer Belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the shutter and around the color registration sensor, and clean or replace it if there is any dirt or abnormality.

- Check the intermediate transfer belt, and clean or replace it if there is any dirt or abnormality.

- Check the connector connection and the wiring between PRCB and the sensor, and repair it when there is any abnormality.

- Check the I/O rate of the sensor by I/O of SD4, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (2/4): 24-O)

- Replace PS62.

- Replace PRCB.

- Replace SD4 and PRCB at a time.

Faulty part isolation DIPSW Control while detached

    - C-4601

Code

C-4601

Classification

Main body: Color registration sensor/Fr (Y) data undetected

Cause

The color registration sensor/Fr (PS60) has detected no data of the Y toner.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board

- Color registration sensor/Fr (PS60)

- Color registration shutter solenoid (SD4)

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Output the test pattern to check the Y toner density or whether the white spot occurs.

- Output the test pattern Number 25 to check the registration mark position in the main scan direction.

- When the registration mark is not output properly, check the writing, developing, and the transfer relation, and repair or replace it when there is any abnormality.

- When the registration mark is output properly, check the connector connection and wiring between PRCB and the color registration sensor, and repair it when there is any abnormality.

Check the color registration shutter solenoid and the color registration sensor, and repair or replace it when there is any abnormality.

- Check the I/O drive of SD4 and repair or replace it when there is any abnormality.

- Replace SD4.

- Replace PS60.

- Replace PRCB.

- Replace the intermediate transfer belt

Faulty part isolation DIPSW Control while detached

    - C-4602

Code

C-4602

Classification

Main body: Color registration sensor/Fr (M) data undetected

Cause

The color registration sensor/Fr (PS60) has detected no data of the M toner.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board

- Color registration sensor/Fr (PS60)

- Color registration shutter solenoid (SD4)

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Output the test pattern to check the M toner density or whether the white spot occurs.

- Output the test pattern Number 25 to check the registration mark position in the main scan direction.

- When the registration mark is not output properly, check the writing, developing, and the transfer relation, and repair or replace it when there is any abnormality.

- When the registration mark is output properly, check the connector connection and wiring between PRCB and the color registration sensor, and repair it when there is any abnormality.

- Check the color registration shutter solenoid and the color registration sensor, and repair or replace it when there is any abnormality. Check the I/O drive of SD4 and repair or replace it when there is any abnormality.

- Replace SD4.

- Replace PS60.

- Replace PRCB.

- Replace the intermediate transfer belt

Faulty part isolation DIPSW Control while detached

    - C-4603

Code

C-4603

Classification

Main body: Color registration sensor/Fr (C) data undetected

Cause

The color registration sensor/Fr (PS60) has detected no data of the C toner.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board

- Color registration sensor/Fr (PS60)

- Color registration shutter solenoid (SD4)

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Output the test pattern to check the C toner density or whether the white spot occurs.

- Output the test pattern Number 25 to check the registration mark position in the main scan direction.

- When the registration mark is not output properly, check the writing, developing, and the transfer relation, and repair or replace it when there is any abnormality.

- When the registration mark is output properly, check the connector connection and wiring between PRCB and the color registration sensor, and repair it when there is any abnormality.

Check the color registration shutter solenoid and the color registration sensor, and repair or replace it when there is any abnormality.

- Check the I/O drive of SD4 and repair or replace it when there is any abnormality.

- Replace SD4.

- Replace PS60.

- Replace PRCB.

- Replace the intermediate transfer belt

Faulty part isolation DIPSW Control while detached

    - C-4604

Code

C-4604

Classification

Main body: Color registration sensor/Fr (K) data undetected

Cause

The color registration sensor/Fr (PS60) has detected no data of the K toner.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board

- Color registration sensor/Fr (PS60)

- Color registration shutter solenoid (SD4)

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Output the test pattern to check the K toner density or whether the white spot occurs.

- Output the test pattern Number 25 to check the registration mark position in the main scan direction.

- When the registration mark is not output properly, check the writing, developing, and the transfer relation, and repair or replace it when there is any abnormality.

- When the registration mark is output properly, check the connector connection and wiring between PRCB and the color registration sensor, and repair it when there is any abnormality.

- Check the color registration shutter solenoid and the color registration sensor, and repair or replace it when there is any abnormality. Check the I/O drive of SD4 and repair or replace it when there is any abnormality.

- Replace SD4.

- Replace PS60.

- Replace PRCB.

- Replace the intermediate transfer belt

Faulty part isolation DIPSW Control while detached

    - C-4611

Code

C-4611

Classification

Main body: Color registration sensor/Rr (Y) data undetected

Cause

The color registration sensor/Rr (PS62) has detected no data of the Y toner.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board

- Color registration sensor/Rr (PS62)

- Color registration shutter solenoid (SD4)

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Output the test pattern to check the Y toner density or whether the white spot occurs.

- Output the test pattern number 25 to check the registration mark position in the main scan direction.

- When the registration mark is not output properly, check the writing, developing, and transfer relations and repair or replace it when there is any abnormality.

- When the registration mark is output properly, check the connector connection and wiring between PRCB and the color registration sensor, and repair it when there is any abnormality.

- Check the color registration sensor and the color registration shutter solenoid, and repair or replace it when there is any abnormality. Check the I/O drive of SD4 and repair or replace it when there is any abnormality.

- Replace SD4.

- Replace PS62.

- Replace PRCB.

- Replace the intermediate transfer belt

Faulty part isolation DIPSW Control while detached

    - C-4612

Code

C-4612

Classification

Main body: Color registration sensor/Rr (M) data undetected

Cause

The color registration sensor/Rr (PS62) has detected no data of the M toner.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board

- Color registration sensor/Rr (PS62)

- Color registration shutter solenoid (SD4)

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Output the test pattern to check the M toner density or whether the white spot occurs.

- Output the test pattern number 25 to check the registration mark position in the main scan direction.

- When the registration mark is not output properly, check the writing, developing, and transfer relations and repair or replace it when there is any abnormality.

- When the registration mark is output properly, check the connector connection and wiring between PRCB and the color registration sensor, and repair it when there is any abnormality.

- Check the color registration sensor and the color registration shutter solenoid, and repair or replace it when there is any abnormality. Check the I/O drive of SD4 and repair or replace it when there is any abnormality.

- Replace SD4.

- Replace PS62.

- Replace PRCB.

- Replace the intermediate transfer belt

Faulty part isolation DIPSW Control while detached

    - C-4613

Code

C-4613

Classification

Main body: Color registration sensor/Rr (C) data undetected

Cause

The color registration sensor/Rr (PS62) has detected no data of the C toner.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board

- Color registration sensor/Rr (PS62)

- Color registration shutter solenoid (SD4)

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Output the test pattern to check the C toner density or whether the white spot occurs.

- Output the test pattern number 25 to check the registration mark position in the main scan direction.

- When the registration mark is not output properly, check the writing, developing, and transfer relations and repair or replace it when there is any abnormality.

- When the registration mark is output properly, check the connector connection and wiring between PRCB and the color registration sensor, and repair it when there is any abnormality.

- Check the color registration sensor and the color registration shutter solenoid, and repair or replace it when there is any abnormality. Check the I/O drive of SD4 and repair or replace it when there is any abnormality.

- Replace SD4.

- Replace PS62.

- Replace PRCB.

- Replace the intermediate transfer belt

Faulty part isolation DIPSW Control while detached

    - C-4614

Code

C-4614

Classification

Main body: Color registration sensor/Rr (K) data undetected

Cause

The color registration sensor/Rr (PS62) has detected no data of the K toner.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board

- Color registration sensor/Rr (PS62)

- Color registration shutter solenoid (SD4)

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Output the test pattern to check the K toner density or whether the white spot occurs.

- Output the test pattern number 25 to check the registration mark position in the main scan direction.

- When the registration mark is not output properly, check the writing, developing, and transfer relations and repair or replace it when there is any abnormality.

- When the registration mark is output properly, check the connector connection and wiring between PRCB and the color registration sensor, and repair it when there is any abnormality.

- Check the color registration sensor and the color registration shutter solenoid, and repair or replace it when there is any abnormality. Check the I/O drive of SD4 and repair or replace it when there is any abnormality.

- Replace SD4.

- Replace PS62.

- Replace PRCB.

- Replace the intermediate transfer belt

Faulty part isolation DIPSW Control while detached

    - C-4631

Code

C-4631

Classification

Main body: Color registration sensor/Fr (Y) abnormality

Cause

The histogram processing of the Y toner has terminated abnormally on the color registration sensor/Fr (PS60).

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board

- Color registration sensor/Fr (PS60)

- Color registration shutter solenoid (SD4)

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Output the test pattern to check whether the uneven density occurs on the Y toner area.

- Check the intermediate transfer belt, and clean or replace it when there is any toner image, damage, or abnormality.

- Check the connector connection and the wiring between PRCB and the color registration sensor, and repair it when there is any abnormality.

- Check the color registration sensor and the color registration shutter solenoid, and repair or replace it when there is any abnormality.

- Check the I/O drive of SD4 and repair or replace it when there is any abnormality.

- Replace SD4.

- Replace PS60.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-4632

Code

C-4632

Classification

Main body: Color registration sensor/Fr (M) abnormality

Cause

The histogram processing of the M toner has terminated abnormally in the color registration sensor/Fr (PS60).

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board

- Color registration sensor/Fr (PS60)

- Color registration shutter solenoid (SD4)

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Output the test pattern to check whether the uneven density occurs on the M toner area.

- Check the intermediate transfer belt, and clean or replace it if there is the toner image or the damage.

- Check the connector connection and the wiring between PRCB and the color registration sensor, and repair it when there is any abnormality.

- Check the color registration sensor and the color registration shutter solenoid, and repair or replace it when there is any abnormality.

- Check the I/O drive of SD4 and repair or replace it when there is any abnormality.

- Replace SD4.

- Replace PS60.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-4633

Code

C-4633

Classification

Main body: Color registration sensor/Fr (C) abnormality

Cause

The histogram processing of the C toner has terminated abnormally in the color registration sensor/Fr (PS60).

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board

- Color registration sensor/Fr (PS60)

- Color registration shutter solenoid (SD4)

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Output the test pattern to check whether the uneven density occurs on the C toner area.

- Check the intermediate transfer belt, and clean or replace it if there is the toner image or the damage.

- Check the connector connection and the wiring between PRCB and the color registration sensor, and repair it when there is any abnormality.

- Check the color registration sensor and the color registration shutter solenoid, and repair or replace it when there is any abnormality.

- Check the I/O drive of SD4 and repair or replace it when there is any abnormality.

- Replace SD4.

- Replace PS60.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-4634

Code

C-4634

Classification

Main body: Color registration sensor/Fr (K) abnormality

Cause

The histogram processing of the K toner has terminated abnormally in the color registration sensor/Fr (PS60).

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board

- Color registration sensor/Fr (PS60)

- Color registration shutter solenoid (SD4)

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Output the test pattern to check whether the uneven density occurs on the K toner area.

- Check the intermediate transfer belt, and clean or replace it if there is the toner image or the damage.

- Check the connector connection and the wiring between PRCB and the color registration sensor, and repair it when there is any abnormality.

- Check the color registration sensor and the color registration shutter solenoid, and repair or replace it when there is any abnormality.

- Check the I/O drive of SD4 and repair or replace it when there is any abnormality.

- Replace SD4.

- Replace PS60.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-4641

Code

C-4641

Classification

Main body: Color registration sensor/Rr (Y) abnormality

Cause

The histogram processing of the Y toner has terminated abnormally in the color registration sensor/Rr (PS62).

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board

- Color registration sensor/Rr (PS62)

- Color registration shutter solenoid (SD4)

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Output the test pattern to check whether the uneven density occurs on the Y toner area.

- Check the intermediate transfer belt, and clean or replace it when there is any toner image, scratch, or damage.

- Check the connector connection and the wiring between PRCB and the color registration sensor, and repair it when there is any abnormality.

- Check the color registration sensor and the color registration shutter solenoid, and repair or replace it when there is any abnormality.

- Check the I/O drive of SD4 and repair or replace it when there is any abnormality.

- Replace SD4.

- Replace PS62.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-4642

Code

C-4642

Classification

Main body: Color registration sensor/Rr (M) abnormality

Cause

The histogram processing of the M toner has terminated abnormally in the color registration sensor/Rr (PS62).

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board

- Color registration sensor/Rr (PS62)

- Color registration shutter solenoid (SD4)

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Output the test pattern to check whether the uneven density occurs on the M toner area.

- Check the intermediate transfer belt, and clean or replace it if there is a the toner image or the damage.

- Check the connector connection and the wiring between PRCB and the color registration sensor, and repair it when there is any abnormality.

- Check the color registration sensor and the color registration shutter solenoid, and repair or replace it when there is any abnormality.

- Check the I/O drive of SD4 and repair or replace it when there is any abnormality.

- Replace SD4.

- Replace PS62.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-4643

Code

C-4643

Classification

Main body: Color registration sensor/Rr (C) abnormality

Cause

The histogram processing of the C toner has terminated abnormally in the color registration sensor/Rr (PS62).

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board

- Color registration sensor/Rr (PS62)

- Color registration shutter solenoid (SD4)

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Output the test pattern to check whether the uneven density occurs on the C toner area.

- Check the intermediate transfer belt, and clean or replace it if there is a the toner image or the damage.

- Check the connector connection and the wiring between PRCB and the color registration sensor, and repair it when there is any abnormality.

- Check the color registration sensor and the color registration shutter solenoid, and repair or replace it when there is any abnormality.

- Check the I/O drive of SD4 and repair or replace it when there is any abnormality.

- Replace SD4.

- Replace PS62.

- Replace PRCB.

Faulty part isolation DIPSW

Control while detached

    - C-4644

Code

C-4644

Classification

Main body: Color registration sensor/Rr (K) abnormality

Cause

The histogram processing of the K toner has terminated abnormally in the color registration sensor/Rr (PS62).

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board

- Color registration sensor/Rr (PS62)

- Color registration shutter solenoid (SD4)

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Output the test pattern to check whether the uneven density occurs on the K toner area.

- Check the intermediate transfer belt, and clean or replace it if there is a the toner image or the damage.

- Check the connector connection and the wiring between PRCB and the color registration sensor, and repair it when there is any abnormality.

- Check the color registration sensor and the color registration shutter solenoid, and repair or replace it when there is any abnormality.

- Check the I/O drive of SD4 and repair or replace it when there is any abnormality.

- Replace SD4.

- Replace PS62.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-4661

Code

C-4661

Classification

Main body: Color registration correction abnormality (Y)

Cause

The color registration correction/Y has terminated abnormally.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board

- Color registration sensor/Fr (PS60), color registration sensor/Rr (PS62)

- Color registration shutter solenoid (SD4)

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the intermediate transfer belt, and clean or replace it when there is any dirt or abnormality.

- Perform the color registration automatic adjustment.

- When the error 2 occurs, acquire the log and analyze it.

- When the error 1 occurs, it means that the color registration error level is not abnormal but not good. In this case, the error is cleared when the machine returns to the normal mode. However it possibly occurs again.

- Check the connector connection and the wiring between PRCB and the color registration sensor, and repair it when there is any abnormality.

- Check the color registration sensor and the color registration shutter solenoid, and repair or replace it when there is any abnormality.

- Check the I/O drive of SD4 and repair or replace it when there is any abnormality.

- Replace SD4.

- Replace PS60 and PS62.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-4662

Code

C-4662

Classification

Main body: Color registration correction abnormality (M)

Cause

The color registration correction/M has terminated abnormally.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board

- Color registration sensor/Fr (PS60), color registration sensor/Rr (PS62)

- Color registration shutter solenoid (SD4)

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the intermediate transfer belt, and clean or replace it when there is any dirt or abnormality.

- Perform the color registration automatic adjustment.

- When the error 2 occurs, acquire the log and analyze it.

- When the error 1 occurs, it means that the color registration error level is not abnormal but not good. In this case, the error is cleared when the machine returns to the normal mode. However it possibly occurs again.

- Check the connector connection and the wiring between PRCB and the color registration sensor, and repair it when there is any abnormality.

- Check the color registration sensor and the color registration shutter solenoid, and repair or replace it when there is any abnormality.

- Check the I/O drive of SD4 and repair or replace it when there is any abnormality.

- Replace SD4.

- Replace PS60 and PS62.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-4663

Code

C-4663

Classification

Main body: Color registration correction abnormality (C)

Cause

The color registration correction/C has terminated abnormally.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board

- Color registration sensor/Fr (PS60), color registration sensor/Rr (PS62)

- Color registration shutter solenoid (SD4)

- Intermediate transfer belt

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the intermediate transfer belt, and clean or replace it when there is any dirt or abnormality.

- Perform the color registration automatic adjustment.

- When the error 2 occurs, acquire the log and analyze it.

- When the error 1 occurs, it means that the color registration error level is not abnormal but not good. In this case, the error is cleared when the machine returns to the normal mode. However it possibly occurs again.

- Check the connector connection and the wiring between PRCB and the color registration sensor, and repair it when there is any abnormality.

- Check the color registration sensor and the color registration shutter solenoid, and repair or replace it when there is any abnormality.

- Check the I/O drive of SD4 and repair or replace it when there is any abnormality.

- Replace SD4.

- Replace PS60 and PS62.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-4705

Code

C-4705

Classification

Main body: Image processing abnormality

Cause

While in the image write, the expansion processing from the memory to the printer does not terminate within a specified period of time. The output from the page memory to the printer does not terminate within a specified period of time. The PVV is not detected within a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer image processing board (PRIPB)

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

1. Replace PRIPB.

Faulty part isolation DIPSW Control while detached

    - C-4709

Code

C-4709

Classification

Main body: Image processing abnormality

Cause

The image processing is not completed within a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer image processing board (PRIPB)

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

1. Replace PRIPB.

Faulty part isolation DIPSW Control while detached

    - C-4713

Code

C-4713

Classification

Main body: Image processing abnormality

Cause

A page memory for printing cannot be secured.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer image processing board (PRIPB)

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

1. Replace PRIPB.

Faulty part isolation DIPSW Control while detached

    - C-4714

Code

C-4714

Classification

Main body: Image processing abnormality/Y

Cause

On APC, the Index sensor output does not change. The index sensor failed to detect the laser because the polygon mirror does not rotate, the position of the index sensor is improper, or the index sensor is defective.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer image processing board (PRIPB)

- Writing unit/Y

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the write unit, and repair or replace it if there is any abnormality.

- Check the connector connection and the wiring between PRIB and the write unit, and repair it when there is any abnormality.

- Replace the write unit.

- Replace PRIPB.

Faulty part isolation DIPSW Control while detached

    - C-4715

Code

C-4715

Classification

Main body: Image processing abnormality/M

Cause

On APC, the Index sensor output does not change. The index sensor failed to detect the laser because the polygon mirror does not rotate, the position of the index sensor is improper, or the index sensor is defective.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer image processing board (PRIPB)

- Writing unit/M

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the write unit, and repair or replace it if there is any abnormality.

- Check the connector connection and the wiring between PRIB and the write unit, and repair it when there is any abnormality.

- Replace the write unit.

- Replace PRIPB.

Faulty part isolation DIPSW Control while detached

    - C-4716

Code

C-4716

Classification

Main body: Image processing abnormality/C

Cause

On APC, the Index sensor output does not change. The index sensor failed to detect the laser because the polygon mirror does not rotate, the position of the index sensor is improper, or the index sensor is defective.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer image processing board (PRIPB)

- Writing unit/C

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the write unit, and repair or replace it if there is any abnormality.

- Check the connector connection and the wiring between PRIB and the write unit, and repair it when there is any abnormality.

- Replace the write unit.

- Replace PRIPB.

Faulty part isolation DIPSW Control while detached

    - C-4717

Code

C-4717

Classification

Main body: Image processing abnormality/K

Cause

On APC, the Index sensor output does not change. The index sensor failed to detect the laser because the polygon mirror does not rotate, the position of the index sensor is improper, or the index sensor is defective.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer image processing board (PRIPB)

- Writing unit/K

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the write unit, and repair or replace it if there is any abnormality.

- Check the connector connection and the wiring between PRIB and the write unit, and repair it when there is any abnormality.

- Replace the write unit.

- Replace PRIPB.

Faulty part isolation DIPSW Control while detached

    - C-4718

Code

C-4718

Classification

Main body: Image processing abnormality/Y

Cause

APC abnormality.

The laser does not turn ON because 12V DC power source for driving laser is not fed, MPC is wrong, or the laser is defective.

Measures to take when an alert occurs

The main body stops immediately to turn OFF the power relay (RL1).

Estimated abnormal parts

- Printer image processing board (PRIPB)

- Writing unit/Y

Correction

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the write unit, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRIB and write unit, and repair it if any abnormality.

- Replace the write unit.

- Replace PRIPB.

Faulty part isolation DIPSW Control while detached

    - C-4719

Code

C-4719

Classification

Main body: Image processing abnormality/M

Cause

APC abnormality.

The laser does not turn ON because 12V DC power source for driving laser is not fed, MPC is wrong, or the laser is defective.

Measures to take when an alert occurs

The main body stops immediately to turn OFF the power relay (RL1).

Estimated abnormal parts

- Printer image processing board (PRIPB)

- Writing unit/M

Correction

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the write unit, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRIB and write unit, and repair it if any abnormality.

- Replace the write unit.

- Replace PRIPB.

Faulty part isolation DIPSW Control while detached

    - C-4720

Code

C-4720

Classification

Main body: Image processing abnormality/C

Cause

APC abnormality.

The laser does not turn ON because 12V DC power source for driving laser is not fed, MPC is wrong, or the laser is defective.

Measures to take when an alert occurs

The main body stops immediately to turn OFF the power relay (RL1).

Estimated abnormal parts

- Printer image processing board (PRIPB)

- Writing unit/C

Correction

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the write unit, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRIB and write unit, and repair it if any abnormality.

- Replace the write unit.

- Replace PRIPB.

Faulty part isolation DIPSW Control while detached

    - C-4721

Code

C-4721

Classification

Main body: Image processing abnormality/K

Cause

APC abnormality.

The laser does not turn ON because 12V DC power source for driving laser is not fed, MPC is wrong, or the laser is defective.

Measures to take when an alert occurs

The main body stops immediately to turn OFF the power relay (RL1).

Estimated abnormal parts

- Printer image processing board (PRIPB)

- Writing unit/K

Correction

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the write unit, and repair it when there is any abnormality.

- Check the connector connection and the wiring between PRIB and write unit, and repair it if any abnormality.

- Replace the write unit.

- Replace PRIPB.

Faulty part isolation DIPSW Control while detached

    - C-4722

Code

C-4722

Classification

Main body: Image processing abnormality/Y

Cause

Connection abnormality between the index board/Y (IDB/Y) or the laser drive board/Y (LDB/Y) and the printer image processing board (PRIPB).

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer image processing board (PRIPB)

- Writing unit/Y

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the write unit, and repair or replace it if there is any abnormality.

- Check the connector connection and the wiring between PRIB and the write unit, and repair it when there is any abnormality.

- Replace the write unit.

- Replace PRIPB.

Faulty part isolation DIPSW Control while detached

    - C-4723

Code

C-4723

Classification

Main body: Image processing abnormality/M

Cause

Connection abnormality between the index board/M (IDB/M) or the laser drive board/M (LDB/M) and the printer image processing board (PRIPB).

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer image processing board (PRIPB)

- Writing unit/M

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the write unit, and repair or replace it if there is any abnormality.

- Check the connector connection and the wiring between PRIB and the write unit, and repair it when there is any abnormality.

- Replace the write unit.

- Replace PRIPB.

Faulty part isolation DIPSW Control while detached

    - C-4724

Code

C-4724

Classification

Main body: Image processing abnormality/C

Cause

Connection abnormality between the index board/C (IDB/C) or the laser drive board/C (LDB/C) and the printer image processing board (PRIPB).

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer image processing board (PRIPB)

- Writing unit/C

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the write unit, and repair or replace it if there is any abnormality.

- Check the connector connection and the wiring between PRIB and the write unit, and repair it when there is any abnormality.

- Replace the write unit.

- Replace PRIPB.

Faulty part isolation DIPSW Control while detached

    - C-4725

Code

C-4725

Classification

Main body: Image processing abnormality/K

Cause

Connection abnormality between the index board/K (IDB/K) or the laser drive board/K (LDB/K) and the printer image processing board (PRIPB).

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer image processing board (PRIPB)

- Writing unit/K

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the write unit, and repair or replace it if there is any abnormality.

- Check the connector connection and the wiring between PRIB and the write unit, and repair it when there is any abnormality.

- Replace the write unit.

- Replace PRIPB.

Faulty part isolation DIPSW Control while detached

    - C-4726

Code

C-4726

Classification

Main body: Image processing abnormality/Y, Image processing abnormality/M, Image processing abnormality/C, Image processing abnormality/ K,

Cause

Installation error of the duct cover.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Duct cover

- Process stand open close sensor (PS30)

- Printer control board (PRCB)

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the condition of the mounting screw of the duct cover and repair it when there is any abnormality.

- Check I/O and the operation of the sensor and repair it when there is any abnormality.

- Check the connector connection and the wiring between the process stand open close sensor (PS30) and the PRCB, and repair it when there is any abnormality.

- Replace PS30.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-4727

Code

C-4727

Classification

Main body: Printer image processing board error

Cause

The power is not supplied, initialization abnormality, device abnormality, or the board is not installed

Measures to take when an alert occurs Main body and options stop immediately. Estimated abnormal parts

- Printer image processing board (PRIPB)

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the installation status of PRIPB, and repair or replace it when there is an abnormality.

- Check the connector connection and the wiring on PRIPB, and repair it when there is an abnormality. (Wiring diagram: Main body (3/4): 1-B), Main body (4/4): 17-C)

- Replace PRIPB.

Faulty part isolation DIPSW Control while detached

    - C-4728

Code

C-4728

Classification

Main body: Printer image processing board error

Cause

The power is not supplied, initialization abnormality, device abnormality, or the board is not installed

Measures to take when an alert occurs Main body and options stop immediately. Estimated abnormal parts

- Printer image processing board (PRIPB)

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the installation status of PRIPB, and repair or replace it when there is an abnormality.

- Check the connector connection and the wiring on PRIPB, and repair it when there is an abnormality. (Wiring diagram: Main body (3/4): 1-B), Main body (4/4): 17-C)

- Replace PRIPB.

Faulty part isolation DIPSW Control while detached

    - C-4840

Code

C-4840

Classification

Main body: Image processing abnormality

Cause

The engine section has kept driving continuously for a specified period of time without producing the image or the process patch.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Printer control firmware

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring on PRCB, and repair it when there is any abnormality.

- Reinstall the printer control firmware

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-4850*

Code

C-4850*

Classification

Main body: Image processing abnormality

Cause

The overall control software accesses an illegal address.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

- Firmware for the overall control

Correction

Deactivate and deactivate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the connector connection and the wiring on OACB, and repair it when there is any abnormality.

- Reinstall the firmware of the overall control.

- Replace OACB.

Faulty part isolation DIPSW Control while detached

  - Solution 10 (C-5001_6000)

    - C-5001

Code

C-5001

Classification

Main body: Communication abnormality

Cause

12 V DC abnormality.

An error signal is detected continuously for a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

      - DC power supply/1 (DCPS1)

      - Printer control board (PRCB)

Correction

Note

      - The message "Please call service" is displayed on the panel.

Solution

- Disconnect CN131 on PRCB to boot the main body, and check the 12V output of CN131-2.

- If there is no 12V output, check the connector connection and the wiring of PRCB and DCPS1. Repair it when there is any abnormality. If no abnormality exists, replace DCPS1.

- Replace PRCB if 12V output performed.

- Replace PRCB and DCPS1 at a time.

Faulty part isolation DIPSW Control while detached

    - C-5002 *

Code

C-5002*

Classification

Main body: Communication abnormality

Cause

24 V DC abnormality.

An error signal is detected continuously for a specified period of time after the sub power switch (SW2) is activated.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

      - Printer control board (PRCB)

      - DC power supply/2 (DCPS2)

      - Each loading of PRCB connection (motor, solenoid and clutch)

Correction

Note

      - The message "Please call service" is displayed on the panel.

Solution

- Disconnect CN100 on PRCB to boot the main body, and check the 24V output of CN100-3 and CN100-4.

- If there is no 24V output, check the connector connection and the wiring of PRCB and DCPS2. Repair it when there is any abnormality. If no abnormality exists, replace DCPS2.

- Replace PRCB if the 24V output is performed.

- Check the loading of the PRCB connection (the motor, the solenoid, and the clutch), and repair or replace it when there is any abnormality.

- Replace PRCB and DCPS2 at a time.

Faulty part isolation DIPSW Control while detached

    - C-5003

Code

C-5003

Classification

Main body: Communication abnormality

Cause

Conveyance drive board (CDB) 24VDC abnormality. An error signal of 24VDC for CDB is detected.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- DC power supply/2 (DCPS2)

- Conveyance drive board (CDB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check that the front door is closed properly, and repair it when there is any abnormality.

- Check the connector connection and the wiring between DCPS2 and CDB, and repair it when there is any abnormality.

- Replace DCPS2.

- Replace CDB.

- Replace DCPS2 and CDB at a same time

Faulty part isolation DIPSW Control while detached

    - C-5006

Code

C-5006

Classification

Main body: Communication abnormality

Cause

24 V DC abnormality.

An error signal is detected after a specified period of time elapses since the sub power switch (SW2) is activated.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

      - Printer control board (PRCB)

      - DC power supply/2 (DCPS2)

      - Each loading of PRCB connection (motor, solenoid and clutch)

Correction

Note

      - The message "Please call service" is displayed on the panel.

Solution

- Disconnect CN100 on PRCB, boot the main body, and check the 24V output of CN100-7.

- If there is no 24V output, check the connector connection and the wiring of PRCB and DCPS2. Repair it when there is any abnormality. If no abnormality exists, replace DCPS2.

- Replace PRCB when the 24V output is performed.

- Check the loading of the PRCB connection (the motor, the solenoid, and the clutch), and repair or replace it when there is any abnormality.

- Replace PRCB and DCPS2 at a time.

Faulty part isolation DIPSW Control while detached

    - C-5007

Code

C-5007

Classification

Main body: Communication abnormality

Cause

24 V DC abnormality.

An error signal is detected after a specified period of time elapses since the sub power switch (SW2) is activated.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

      - Printer control board (PRCB)

      - DC power supply/2 (DCPS2)

      - Each loading of PRCB connection (motor, solenoid and clutch)

Correction

Note

      - The message "Please call service" is displayed on the panel.

Solution

- Check that the front door is closed properly, and repair it when there is any abnormality.

- Check the operation of the front door switch (MS1), and repair it when there is any abnormality.

- Disconnect CN101 on PRCB to boot the main body, and check the 24V output of CN101-1.

- If there is no 24V output, check the connector connection and the wiring of PRCB and DCPS2. Repair it when there is any abnormality. If no abnormality exists, replace DCPS2.

- Replace PRCB when the 24V output is performed.

- Check the loading of the PRCB connection (the motor, the solenoid, and the clutch), and repair or replace it when there is any abnormality.

- Replace PRCB and DCPS2 at a time.

Faulty part isolation DIPSW Control while detached

    - C-5010

Code

C-5010

Classification

Main body: Communication abnormality

Cause

A line status error or a checksum error occurred between the overall control board (OACB), the printer image processing board (PRIPB) and the printer control board (PRCB) is detected and sending a retransmission request does not recover the error. The recovery failed even after the requested retransmission is performed. A sufficient space cannot be obtained in the circular buffer within a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Overall control board (OACB)

Correction Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring between PRIPB and OACB, and repair it when there is any abnormality.

- Replace PRCB.

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW Control while detached

    - C-5301*

Code

C-5301*

Classification

Main body: Fan abnormality

Cause

When the power supply cooling fan/3 (FM4) is active, an error detection signal of FM4 is detected continuously for a specified period of time. An error detection signal is detected continuously while the power is fed again several times.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Power supply cooling fan/3 (FM4)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Rotate the fan or I/O manually, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (1/4): 21-E)

- Check the connector connection and the wiring between PRCB and the fan, and repair it when there is any abnormality.

- Replace FM4.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-5305*

Code

C-5305*

Classification

Main body: Fan abnormality

Cause

When the power supply cooling fan/4 (FM5) is active, an error detection signal of FM5 is detected continuously for a specified period of time. An error detection signal is detected continuously while the power is fed again several times.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Power supply cooling fan/4 (FM5)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Rotate the fan or I/O manually, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (1/4): 21-E)

- Check the connector connection and the wiring between PRCB and the fan, and repair it when there is any abnormality.

- Replace FM5.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-5307*

Code

C-5307

Classification

Main body: Fan abnormality

Cause

When the power supply cooling fan/2 (FM3) is active, an error detection signal of FM3 is detected continuously for a specified period of time. An error detection signal is detected continuously while the power is fed again several times.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Power supply cooling fan/2 (FM3)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Rotate the fan or I/O manually, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (1/4): 12-L)

- Check the connector connection and the wiring between PRCB and the fan, and repair it when there is any abnormality.

- Replace FM3.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-5309*

Code

C-5309*

Classification

Main body: Fan abnormality

Cause

When the PH cooling fan (FM18) is active, an error detection signal of FM18 is detected continuously for a specified period of time. An error detection signal is detected continuously while the power is fed again several times.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- PH cooling fan (FM18)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Rotate the fan by I/O or manually, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (1/4):15-L)

- Check the connector connection and the wiring between PRCB and the fan, and repair it when there is any abnormality.

- Replace FM18.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-5310*

Code

C-5310*

Classification

Main body: Fan abnormality

Cause

When the exhaust fan/Rr (FM14) is active, an error detection signal of FM14 is detected continuously for a specified period of time. An error detection signal is detected continuously while the power is fed again several times.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Exhaust fan/Rr (FM14)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Rotate the fan by I/O or manually, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (1/4):13-L)

- Check the connector connection and the wiring between PRCB and the fan, and repair it when there is any abnormality.

- Replace FM14.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-5311*

Code

C-5311*

Classification

Main body: Fan abnormality

Cause

When the power supply cooling fan/3 (FM4) is active, an error detection signal of FM4 is detected continuously for a specified period of time. An error detection signal is detected continuously while the power is fed again several times.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Power supply cooling fan/3 (FM4)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Rotate the fan or I/O manually, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (1/4): 21-E)

- Check the connector connection and the wiring between PRCB and the fan, and repair it when there is any abnormality.

- Replace FM4.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-5401

Code

C-5401

Classification

Main body: Fan abnormality

Cause

The fan lock detection for the overall control CPU cooling fan on the overall control board (OACB) is monitored and an error is detected. After it is monitored again, the error is detected.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Rotate the fan by hand, and repair or replace it when there is any abnormality.

- Check that the fan cable is inserted into CN2301 of OACB.

- Replace OACB when there is no abnormality with the insertion condition of the fan cable. (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW Control while detached

    - C-5402

Code

C-5402

Classification

Main body: Fan abnormality

Cause

When the image processing board ASIC cooling fan on the printer image processing board (PRIPB) is active, an error detection signal of the fan is detected continuously for a specified period of time. An error detection signal is detected continuously while the power is fed again several times.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer image processing board (PRIPB)

- Overall control board (OACB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Rotate the fan by hand, and repair it when there is any abnormality.

- Disconnect CN405 on PRIPB to boot the main body, and check the 12V output of CN405-1.

- Disconnect CN405 on PRIPB to boot the main body, and check the voltage level of CN405-3.

- If there is the 3.3V output and no abnormality with the voltage level, replace PRIPB.

- When there is no abnormality with the voltage level, replace OACB. (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW Control while detached

    - C-5403

Code

C-5403

Classification

Main body: Fan abnormality

Cause

When the image processing cooling fan (FM1) is active, an error detection signal of FM1 is detected continuously for a specified period of time. An error detection signal is detected continuously while the power is fed again several times.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

- Image processing cooling fan (FM1)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Rotate the fan or I/O manually, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (3/4): 21-L)

- Check the connector connection and the wiring between OACB and the fan, and repair it when there is any abnormality.

- Replace FM1.

Faulty part isolation DIPSW Control while detached

    - C-5404

Code

C-5404

Classification

Main body: Fan abnormality

Cause

When the power supply cooling fan/1 (FM2) is active, an error detection signal of FM2 is detected continuously for a specified period of time. An error detection signal is detected continuously while the power is fed again several times.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

- Power supply cooling fan/1 (FM2)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Rotate the fan or I/O manually, and repair or replace it when there is any abnormality. (Wiring diagram: Main body (3/4): 21-L)

- Check the connector connection and the wiring between OACB and the fan, and repair it when there is any abnormality.

- Replace FM2.

Faulty part isolation DIPSW Control while detached

  - Solution 11 (C-6001_7000)

    - C-6801

Code

C-6801

Classification

Main body: Communication error

Cause

The communication between the over all control board (OACB) and the operation board/1 (OB1) does not start within a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

- Operation board/1 (OB1)

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the connector connection and the wiring on OACB and OB1, and repair it when there is any abnormality.

- Replace OB1.

- Replace OACB.

Faulty part isolation DIPSW Control while detached

    - C-6802

Code

C-6802

Classification

Main body: I2ROM incorrect machine type abnormality

Cause

The I2ROM for the different machine is written.

* No SC Count

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

      - Firmware for the overall control

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

      - The message "Please turn on power again" is displayed on the touch panel.

Solution

1. Reinstall the firmware of the overall control.

Faulty part isolation DIPSW Control while detached

  - Solution 12 (C-7001_9999)

    - C-7001

Code

C-7001

Classification

Main body: Communication error

Cause

Communication error.

Measures to take when an alert occurs

The main body and the option stop immediately.

Estimated abnormal parts

- Printer control board (PRCB)

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the connector connection and the wiring between the main body and the post processing machine, and repair it if any abnormality.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

  - Solution 13 (C-C001_C200)

    - C-C101

Code

C-C101

Classification

Main body: Communication abnormality

Cause

When the sub power switch (SW2) becomes active, a response from the printer control board (PRCB) has not received within a specified period of time.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer image processing board (PRIPB)

- Printer control board (PRCB)

- Overall control board (OACB)

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the connector connection and the wiring of PRIPB, PRCB and OACB, and repair it when there is any abnormality.

- Replace PRIPB.

- Replace PRCB.

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

- Replace PRIPB, PRCB and OACB at a time

Faulty part isolation DIPSW Control while detached

    - C-C102

Code

C-C102

Classification

Main body: Communication abnormality

Cause

Communication error of the printer control board (PRCB)

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer image processing board (PRIPB)

- Printer control board (PRCB)

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the connector connection and the wiring among PRIPB, PRCB and OACB and repair it when there is any abnormality.

- Replace PRIPB.

- Replace PRCB.

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW Control while detached

    - C-C104

Code

C-C104

Classification

Main body: ISW abnormality

Cause

When the sub power switch (SW2) becomes active, ISW unwritten area is detected in the printer control program.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control firmware

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Reinstalling PRCB firmware

- Check the I/F connector connection and the wiring for ISW on RBU, and repair it when there is any abnormality.

- Check the connector connection and the wiring between RBU and OACB, and repair it when there is any abnormality.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-C120

Code

C-C120

Classification

Main body: ISW abnormality

Cause

Printer control board (PRCB) firmware abnormality

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control firmware

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Reinstall PRCB firmware

- Check the I/F connector connection and the wiring for ISW on RBU, and repair it when there is any abnormality.

- Check the connector connection and the wiring between RBU and OACB, and repair it when there is any abnormality.

- Replace PRCB.

Faulty part isolation DIPSW Control while detached

    - C-C121

Code

C-C121

Classification

Main body: Overall control ISW unwritten

Cause

When the sub power switch (SW2) becomes active, the ISW unwritten area is detected in the overall control program.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Firmware for the overall control

- SSD (SSD)

- Overall control board (OACB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Reinstall the firmware of the overall control.

- Check the connector connection and the wiring on OACB, and repair it when there is any abnormality.

- Replace SSD (Refer to "G.3.2.9.SSD (SSD)")

Note

  - For how to get or replace the SSD, contact the service manager of Konica Minolta.

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW Control while detached

    - C-C131

Code

C-C131

Classification Main body: - Cause

Contact the service manager of KM.

Measures to take when an alert occurs

The main body and the option stop immediately.

Estimated abnormal parts

Correction

Note

      - The message "Please call service" is displayed on the panel.

Solution

1.

Faulty part isolation DIPSW Control while detached

    - C-C136

Code

C-C136

Classification

Main body: The version of CommonAPI does not match

Cause

When the version differs between the Common API for the overall control and the Common API for the IC controller.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the connector connection and the wiring on OACB, and repair it when there is any abnormality.

- Reinstall the firmware of the overall control (I1) and the IC controller (P).

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW Control while detached

    - C-C140

Code

C-C140

Classification

Main body: -

Cause

"0" is selected on DIPSW52-2.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

•

Correction

Note

      - The message "Please call service" is displayed on the panel.

Solution

1. Change DIPSW52-2 to "1". Faulty part isolation DIPSW Control while detached

    - C-C150

Code

C-C150

Classification

Main body: Protection chip combination error

Cause

      - When DIPSW40-7=1, the protection chip/1 (CP1) is not installed.

      - A protection chip other than the CP1 is installed.

Measures to take when alert occurs Main body and options stop immediately. Estimated abnormal parts

      - Protection chip/1 (CP1)

      - A protection chip other than the protection chip/1 (CP1).

Correction

Note

      - The message "Please call service" is displayed on the panel.

Solution

1. Check the installed protection chips that are equipped, and repair it when it is not correct.

Faulty part isolation DIPSW Control while detached

    - C-C151

Code

C-C151

Classification

Main body: Memory size error

Cause

If the memory size is less than the minimum value that is prescribed in each controller mode, do not boot the controller firmware and regard it as the malfunction status.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- DIMM/1 (DIMM1)

- DIMM/2 (DIMM2)

- DIMM/3 (DIMM3)

- DIMM/4 (DIMM4)

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

1. Check the relation between the memory size and the controller mode, and repair it if the relation is improper.

Faulty part isolation DIPSW Control while detached

    - C-C170

Code

C-C170

Classification

Main body: -

Cause

"1" is selected on DIPSW49-1.

Measures to take when an alert occurs

The main body and the option stop immediately.

Estimated abnormal parts

•

Correction

Note

      - The message "Please call service" is displayed on the panel.

Solution

1. Select "0" on DIPSW49-1. Faulty part isolation DIPSW Control while detached

  - Solution 14 (C-C201_E100)

    - C-D010

Code

C-D010

Classification

Main body: During or after the HDD deletion

Cause

The HDD overwriting and deleting is being executed, or the HDD overwriting and deleting has been executed.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Hard disk/1 (HDD1)

- Hard disk /2 (HDD2)

- Hard disk/3 (HDD3)

- Hard disk/4 (HDD4)

Correction

Press the help key to activate and deactivate the sub power switch (SW2) of the main body.

Note

- When you isolate the faulty part, the limited use is available.

- The message "Please call service" and "Press [Help] to disengage" are displayed alternately on the touch panel.

- When you press the help key, the message "Please turn on power again" is displayed on the touch panel.

Solution

1. Report it to KM.

Faulty part isolation DIPSW

DIPSW18-7

Control while detached

HDD unusable (HDD disconnected)

*When connected to IC-602, do not detach.

    - C-D020

Code

C-D020

Classification

Main body: HDD abnormality

Cause

An error occurs with the HDD.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

- Hard disk/1 (HDD1)

- Hard disk /2 (HDD2)

- Hard disk/3 (HDD3)

- Hard disk/4 (HDD4)

Correction

Press the help key to activate and deactivate the sub power switch (SW2) of the main body.

Note

- When you isolate the faulty part, the limited use is available.

- The message "Please call service" and "Press [Help] to disengage" are displayed alternately on the touch panel.

- When you press the help key, the message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the firmware of the overall control (I) and the controller (P), and reinstall the firmware when the version is different.

- Execute bad sectors check and recovery in the output check I/O 99-03.

- Check the connection between each HDD, and repair it when there is any abnormality.

- Format the HDD (Refer to I.4.15.2.Format HDD All Data)

- When UK-104 is not mounted, replace HDD1, HDD2 and HDD3 (Refer to "I.4.7.8.Hard disk replacing procedure") When UK-104 is mounted, perform steps 6 to 9.

- Change DIPSW40-7 to "0".

- OFF/ON of the sub power switch (SW2)

- Check if the malfunction code C-D0E1 occurs at the start.

<When C-D0E1 occurs>

  - When either of HDD1, HDD2, or HDD3 is displayed on the touch panel, replace HDD1, HDD2, and HDD3. (Refer to "I.4.7.8.Hard disk replacing procedure")

  - When HDD4 is displayed on the touch panel, replace HDD4. (Refer to "I.4.7.8.Hard disk replacing procedure")

<When C-D0E1 does not occur>

  - Replace HDD4 (Refer to "I.4.7.8.Hard disk replacing procedure")

- Change DIPSW40-7 to "1".

- Replace the other HDD which is not replaced yet. (Refer to "I.4.7.8.Hard disk replacing procedure")

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW

DIPSW18-7

Control while detached

HDD unusable (HDD disconnected)

*When connected to IC-602, do not detach.

    - C-D030

Code

C-D030

Classification

Main body: HDD unformatted

Cause

The HDD is not formatted.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

- Hard disk/1 (HDD1)

- Hard disk /2 (HDD2)

- Hard disk/3 (HDD3)

- Hard disk/4 (HDD4)

Correction

Press the help key to activate and deactivate the sub power switch (SW2) of the main body.

Note

- When you isolate the faulty part, the limited use is available.

- The message "Please call service" and "Press [Help] to disengage" are displayed alternately on the touch panel.

- When you press the help key, the message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the firmware of the overall control (I) and the controller (P), and reinstall the firmware when the version is different.

- Execute bad sectors check and recovery in the output check I/O 99-03.

- Check the connection between each HDD, and repair it when there is any abnormality.

- Format the HDD (Refer to I.4.15.2.Format HDD All Data)

- When UK-104 is not mounted, replace HDD1, HDD2 and HDD3 (Refer to "I.4.7.8.Hard disk replacing procedure") When UK-104 is mounted, perform steps 6 to 9.

- Change DIPSW40-7 to "0".

- OFF/ON of the sub power switch (SW2)

- Check if the malfunction code C-D0E1 occurs at the start.

<When C-D0E1 occurs>

  - When either of HDD1, HDD2, or HDD3 is displayed on the touch panel, replace HDD1, HDD2, and HDD3. (Refer to "I.4.7.8.Hard disk replacing procedure")

  - When HDD4 is displayed on the touch panel, replace HDD4. (Refer to "I.4.7.8.Hard disk replacing procedure")

<When C-D0E1 does not occur>

  - Replace HDD4 (Refer to "I.4.7.8.Hard disk replacing procedure")

- Change DIPSW40-7 to "1".

- Replace the other HDD which is not replaced yet. (Refer to "I.4.7.8.Hard disk replacing procedure")

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW

DIPSW18-7

Control while detached

HDD unusable (HDD disconnected)

*When connected to IC-602, do not detach.

    - C-D040

Code

C-D040

Classification

Main body: RAID abnormality

Cause

An error occurs on the RAID.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

- Hard disk/1 (HDD1)

- Hard disk /2 (HDD2)

- Hard disk/3 (HDD3)

- Hard disk/4 (HDD4)

Correction

Press the help key to activate and deactivate the sub power switch (SW2) of the main body.

Note

- When you isolate the faulty part, the limited use is available.

- The message "Please call service" and "Press [Help] to disengage" are displayed alternately on the touch panel.

- When you press the help key, the message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the firmware of the overall control (I) and the controller (P), and reinstall the firmware when the version is different.

- Execute bad sectors check and recovery in the output check I/O 99-03.

- Check the connection between each HDD, and repair it when there is any abnormality.

- Format the HDD (Refer to I.4.15.2.Format HDD All Data)

- When UK-104 is not mounted, replace HDD1, HDD2 and HDD3 (Refer to "I.4.7.8.Hard disk replacing procedure") When UK-104 is mounted, perform steps 6 to 9.

- Change DIPSW40-7 to "0".

- OFF/ON of the sub power switch (SW2)

- Check if the malfunction code C-D0E1 occurs at the start.

<When C-D0E1 occurs>

  - When either of HDD1, HDD2, or HDD3 is displayed on the touch panel, replace HDD1, HDD2, and HDD3. (Refer to "I.4.7.8.Hard disk replacing procedure")

  - When HDD4 is displayed on the touch panel, replace HDD4. (Refer to "I.4.7.8.Hard disk replacing procedure")

<When C-D0E1 does not occur>

  - Replace HDD4 (Refer to "I.4.7.8.Hard disk replacing procedure")

- Change DIPSW40-7 to "1".

- Replace the other HDD which is not replaced yet. (Refer to "I.4.7.8.Hard disk replacing procedure")

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW

DIPSW18-7

Control while detached

HDD unusable (HDD disconnected)

*When connected to IC-602, do not detach.

    - C-D050

Code

C-D050

Classification

Main body: RAID access abnormality

Cause

An error occurs when the RAID is accessed.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

- Hard disk/1 (HDD1)

- Hard disk /2 (HDD2)

- Hard disk/3 (HDD3)

- Hard disk/4 (HDD4)

Correction

Press the help key to activate and deactivate the sub power switch (SW2) of the main body.

Note

- When you isolate the faulty part, the limited use is available.

- The message "Please call service" and "Press [Help] to disengage" are displayed alternately on the touch panel.

- When you press the help key, the message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the firmware of the overall control (I) and the controller (P), and reinstall the firmware when the version is different.

- Execute bad sectors check and recovery in the output check I/O 99-03.

- Check the connection between each HDD, and repair it when there is any abnormality.

- Format the HDD (Refer to I.4.15.2.Format HDD All Data)

- When UK-104 is not mounted, replace HDD1, HDD2 and HDD3 (Refer to "I.4.7.8.Hard disk replacing procedure") When UK-104 is mounted, perform steps 6 to 9.

- Change DIPSW40-7 to "0".

- OFF/ON of the sub power switch (SW2)

- Check if the malfunction code C-D0E1 occurs at the start.

<When C-D0E1 occurs>

  - When either of HDD1, HDD2, or HDD3 is displayed on the touch panel, replace HDD1, HDD2, and HDD3. (Refer to "I.4.7.8.Hard disk replacing procedure")

  - When HDD4 is displayed on the touch panel, replace HDD4. (Refer to "I.4.7.8.Hard disk replacing procedure")

<When C-D0E1 does not occur>

  - Replace HDD4 (Refer to "I.4.7.8.Hard disk replacing procedure")

- Change DIPSW40-7 to "1".

- Replace the other HDD which is not replaced yet. (Refer to "I.4.7.8.Hard disk replacing procedure")

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW

DIPSW18-7

Control while detached

HDD unusable (HDD disconnected)

*When connected to IC-602, do not detach.

    - C-D060

Code

C-D060

Classification

Main body: Controller area abnormality (SSD)

Cause

An error occurs with the SSD controller area (Controller error).

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

- SSD

Correction

Press the help key to activate and deactivate the sub power switch (SW2) of the main body.

Note

- When you isolate the faulty part, the limited use is available.

- The message "Please call service" and "Press [Help] to disengage" are displayed alternately on the touch panel.

- When you press the help key, the message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the connection between SSD and OACB, and repair it when there is any abnormality.

- Reinstall the firmware of the controller.

- Replace SSD (Refer to "G.3.2.9.SSD (SSD)")

Note

  - For how to get or replace the SSD (SSD), contact the service manager of Konica Minolta.

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW Control while detached

    - C-D070

Code

C-D070

Classification

Main body: Controller abnormality

Cause

The controller error except C-D020 to C-D060

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

- Hard disk/1 (HDD1)

- Hard disk /2 (HDD2)

- Hard disk/3 (HDD3)

- Hard disk/4 (HDD4)

Correction

Press the help key to activate and deactivate the sub power switch (SW2) of the main body.

Note

- When you isolate the faulty part, the limited use is available.

- The message "Please call service" and "Press [Help] to disengage" are displayed alternately on the touch panel.

- When you press the help key, the message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the firmware of the overall control (I) and the controller (P), and reinstall the firmware when the version is different.

- Execute bad sectors check and recovery in the output check I/O 99-03.

- Check the connection between each HDD, and repair it when there is any abnormality.

- Format the HDD (Refer to I.4.15.2.Format HDD All Data)

- When UK-104 is not mounted, replace HDD1, HDD2 and HDD3 (Refer to "I.4.7.8.Hard disk replacing procedure") When UK-104 is mounted, perform steps 6 to 9.

- Change DIPSW40-7 to "0".

- OFF/ON of the sub power switch (SW2)

- Check if the malfunction code C-D0E1 occurs at the start.

<When C-D0E1 occurs>

  - When either of HDD1, HDD2, or HDD3 is displayed on the touch panel, replace HDD1, HDD2, and HDD3. (Refer to "I.4.7.8.Hard disk replacing procedure")

  - When HDD4 is displayed on the touch panel, replace HDD4. (Refer to "I.4.7.8.Hard disk replacing procedure")

<When C-D0E1 does not occur>

  - Replace HDD4 (Refer to "I.4.7.8.Hard disk replacing procedure")

- Change DIPSW40-7 to "1".

- Replace the other HDD which is not replaced yet. (Refer to "I.4.7.8.Hard disk replacing procedure")

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW

DIPSW18-7

Control while detached

HDD unusable (HDD disconnected)

*When connected to IC-602, do not detach.

    - C-D080

Code

C-D080

Classification

Main body: Common API related HDD file crushed

Cause

The Common API related file on the HDD is damaged.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

- Hard disk/1 (HDD1)

- Hard disk /2 (HDD2)

- Hard disk/3 (HDD3)

Correction

Press the help key to activate and deactivate the sub power switch (SW2) of the main body.

Note

- When you isolate the faulty part, the limited use is available.

- The message "Please call service" and "Press [Help] to disengage" are displayed alternately on the touch panel.

- When you press the help key, the message "Please turn on power again" is displayed on the touch panel.

Solution

- Execute bad sectors check and recovery in the output check I/O 99-03.

- Check the connection between each HDD, and repair it when there is any abnormality.

- Format the HDD (Refer to I.4.15.2.Format HDD All Data)

- Replace HDD1, HDD2 and HDD3 (Refer to "I.4.7.8.Hard disk replacing procedure")

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW

DIPSW18-7

Control while detached

HDD unusable (HDD disconnected)

*When connected to IC-602, do not detach.

    - C-D0C0

Code

C-D0C0

Classification

Main body: HDD parameter abnormality

Cause

An error occurs with the HDD parameter.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

- Hard disk/1 (HDD1)

- Hard disk /2 (HDD2)

- Hard disk/3 (HDD3)

Correction

Press the help key to activate and deactivate the sub power switch (SW2) of the main body.

Note

- When you isolate the faulty part, the limited use is available.

- The message "Please call service" and "Press [Help] to disengage" are displayed alternately on the touch panel.

- When you press the help key, the message "Please turn on power again" is displayed on the touch panel.

Solution

- Initialize the file system by the I/O 99-92 output check.

- Check the connection between each HDD, and repair it when there is any abnormality.

- Format the HDD (Refer to I.4.15.2.Format HDD All Data)

- Replace HDD1, HDD2 and HDD3 (Refer to "I.4.7.8.Hard disk replacing procedure")

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW

DIPSW18-7

Control while detached

HDD unusable (HDD disconnected)

*When connected to IC-602, do not detach.

    - C-D0D0

Code

C-D0D0

Classification

Main body: HDD connection port unmatched

Cause

The HDD is connected incorrectly.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

- Hard disk/1 (HDD1)

- Hard disk /2 (HDD2)

- Hard disk/3 (HDD3)

- Hard disk/4 (HDD4)

Correction

Press the help key to activate and deactivate the sub power switch (SW2) of the main body.

Note

- When you isolate the faulty part, the limited use is available.

- The message "Please call service" and "Press [Help] to disengage" are displayed alternately on the touch panel.

- When you press the help key, the message "Please turn on power again" is displayed on the touch panel.

Solution

- Check that each HDD is connected to the appropriate port, and repair it when there is any abnormality.

- Format the HDD (Refer to I.4.15.2.Format HDD All Data)

- Replace HDD (Refer to "I.4.7.8.Hard disk replacing procedure")

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW

DIPSW18-7

Control while detached

HDD unusable (HDD disconnected)

*When connected to IC-602, do not detach.

    - C-D0D1

Code

C-D0D1

Classification

Main body: HDD setting abnormality

Cause

HDD configuration error.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

- Hard disk/1 (HDD1)

- Hard disk /2 (HDD2)

- Hard disk/3 (HDD3)

- Hard disk/4 (HDD4)

Correction

Press the help key to activate and deactivate the sub power switch (SW2) of the main body.

Note

- When you isolate the faulty part, the limited use is available.

- The message "Please call service" and "Press [Help] to disengage" are displayed alternately on the touch panel.

- When you press the help key, the message "Please turn on power again" is displayed on the touch panel.

Solution

- Check that the HDD configuration matches the DIPSW setting, and repair it when there is any abnormality.

<Proper DIPSW setting>

  - When the number of HDD is 0: DIPSW36-6=0, DIPSW2-0=0

  - When the number of HDD is 3: DIPSW36-6=0, DIPSW2-0=1

  - When the number of HDD is 4: DIPSW36-6=1, DIPSW2-0=1

- Deactivate and activate the sub power switch (SW2) and the main power switch (SW1).

- Check the connection status of the protection chip/1 (CP1) and the protection chip/2 (CP2), and repair it when there is any abnormality.

<Proper connection>

  - For 3 HDDs: Remove the protection chip/2 (CP2)

  - For 4 HDDs: Connect the protection chip/2 (CP2)

- Check the connection between each HDD, and repair it when there is any abnormality.

- Format the HDD (Refer to I.4.15.2.Format HDD All Data)

- Replace HDD (Refer to "I.4.7.8.Hard disk replacing procedure")

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW

DIPSW18-7

Control while detached

HDD unusable (HDD disconnected)

*When connected to IC-602, do not detach.

    - C-D0E1

Code

C-D0E1

Classification

Main body: HDD abnormality

Cause

The HDD1, HDD2, HDD3, or HDD4 is disconnected, or one or more HDDs are damaged. Alternatively, an HDD that KM does not specify is installed.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

- Hard disk/1 (HDD1)

- Hard disk /2 (HDD2)

- Hard disk/3 (HDD3)

- Hard disk/4 (HDD4)

Correction

Press the help key to activate and deactivate the sub power switch (SW2) of the main body.

Note

- When you isolate the faulty part, the limited use is available.

- The message "Please call service" and "Press [Help] to disengage" are displayed alternately on the touch panel.

- When you press the help key, the message "Please turn on power again" is displayed on the touch panel.

Solution

- Execute bad sectors check and recovery in the output check I/O 99-03.

- Check the connection between each HDD, and repair it when there is any abnormality.

- Format the HDD (Refer to I.4.15.2.Format HDD All Data)

- When either of HDD1, HDD2, or HDD3 is displayed on the touch panel, replace HDD1, HDD2, and HDD3. (Refer to "I.4.7.8.Hard disk replac- ing procedure")

When HDD4 is displayed on the touch panel, replace HDD4. (Refer to "I.4.7.8.Hard disk replacing procedure")

- Replace the other HDD which is not replaced yet. (Refer to "I.4.7.8.Hard disk replacing procedure")

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW

DIPSW18-7

Control while detached

HDD unusable (HDD disconnected)

*When connected to IC-602, do not detach.

    - C-D0E3

Code

C-D0E3

Classification

Main body: HDD1 read or write abnormality

Cause

The reading and writing operation to the HDD1 is failed.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

- Hard disk/1 (HDD1)

Correction

Press the help key to activate and deactivate the sub power switch (SW2) of the main body.

Note

- When you isolate the faulty part, the limited use is available.

- The message "Please call service" and "Press [Help] to disengage" are displayed alternately on the touch panel.

- When you press the help key, the message "Please turn on power again" is displayed on the touch panel.

Solution

- Execute bad sectors check and recovery in the output check I/O 99-03.

- Check the connection between each HDD, and repair it when there is any abnormality.

- Replace HDD1, HDD2 and HDD3 (Refer to "I.4.7.8.Hard disk replacing procedure")

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW

DIPSW18-7

Control while detached

HDD unusable (HDD disconnected)

*When connected to IC-602, do not detach.

    - C-D0E4

Code

C-D0E4

Classification

Main body: HDD2 read or write abnormality

Cause

The reading and writing operation to the HDD2 is failed.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

- Hard disk /2 (HDD2)

Correction

Press the help key to activate and deactivate the sub power switch (SW2) of the main body.

Note

- When you isolate the faulty part, the limited use is available.

- The message "Please call service" and "Press [Help] to disengage" are displayed alternately on the touch panel.

- When you press the help key, the message "Please turn on power again" is displayed on the touch panel.

Solution

- Execute bad sectors check and recovery in the output check I/O 99-03.

- Check the connection between each HDD, and repair it when there is any abnormality.

- Replace HDD1, HDD2 and HDD3 (Refer to "I.4.7.8.Hard disk replacing procedure")

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW

DIPSW18-7

Control while detached

HDD unusable (HDD disconnected)

*When connected to IC-602, do not detach.

    - C-D0E5

Code

C-D0E5

Classification

Main body: HDD3 read or write abnormality

Cause

The reading and writing operation to the HDD3 is failed.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

- Hard disk/3 (HDD3)

Correction

Press the help key to activate and deactivate the sub power switch (SW2) of the main body.

Note

- When you isolate the faulty part, the limited use is available.

- The message "Please call service" and "Press [Help] to disengage" are displayed alternately on the touch panel.

- When you press the help key, the message "Please turn on power again" is displayed on the touch panel.

Solution

- Execute bad sectors check and recovery in the output check I/O 99-03.

- Check the connection between each HDD, and repair it when there is any abnormality.

- Replace HDD1, HDD2 and HDD3 (Refer to "I.4.7.8.Hard disk replacing procedure")

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW

DIPSW18-7

Control while detached

HDD unusable (HDD disconnected)

*When connected to IC-602, do not detach.

    - C-D0E8

Code

C-D0E8

Classification

Main body: HDD read or write abnormality

Cause

The reading and writing operations to the HDD is failed.

Measures to take when an alert occurs

Main body and options stop immediately.

Estimated abnormal parts

- Overall control board (OACB)

- Hard disk/1 (HDD1)

- Hard disk/2 (HDD2)

- Hard disk/3 (HDD3)

Correction

Press the help key to activate and deactivate the sub power switch (SW2) of the main body.

Note

- When you isolate the faulty part, the limited use is available.

- The message "Please call service" and "Press [Help] to disengage" are displayed alternately on the touch panel.

- When you press the help key, the message "Please turn on power again" is displayed on the touch panel.

Solution

- Execute bad sectors check and recovery in the output check I/O 99-92.

- Check the connection between each HDD, and repair it when there is any abnormality.

- Execute Format HDD

- Replace HDD1, HDD2, and HDD3 at the same time.(Refer to "I.4.7.8.Hard disk replacing procedure")

- Replace OACB. (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW

DIPSW18-7

Control while detached

HDD unusable (HDD disconnected)

*When connected to IC-602, do not detach.

    - C-D0EF

Code

C-D0EF

Classification

Main body: HDD verify abnormality

Cause

The HDD verify check detects an error.

* No SC Count

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

      - Overall control board (OACB)

      - Hard disk/1 (HDD1)

      - Hard disk /2 (HDD2)

      - Hard disk/3 (HDD3)

Correction

Press the help key to activate and deactivate the sub power switch (SW2) of the main body.

Note

      - When you isolate the faulty part, the limited use is available.

      - The message "Please call service" and "Press [Help] to disengage" are displayed alternately on the touch panel.

      - When you press the help key, the message "Please turn on power again" is displayed on the touch panel.

Solution

- Execute bad sectors check and recovery in the output check I/O 99-03.

- Check the connection between each HDD, and repair it when there is any abnormality.

- Replace HDD1, HDD2 and HDD3 (Refer to "I.4.7.8.Hard disk replacing procedure")

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW

DIPSW18-7

Control while detached

HDD unusable (HDD disconnected)

*When connected to IC-602, do not detach.

    - C-D0F0

Code

C-D0F0

Classification

Main body: HDD Lock abnormality

Cause

HDD Lock abnormality under the enhanced security

The HDD is locked because the HDD lock password is incorrect or not registered, or because the enhanced security mode is not active.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

- Hard disk/1 (HDD1)

- Hard disk /2 (HDD2)

- Hard disk/3 (HDD3)

- Hard disk/4 (HDD4)

Correction

Press the help key to activate and deactivate the sub power switch (SW2) of the main body.

Note

- When you isolate the faulty part, the limited use is available.

- The message "Please call service" and "Press [Help] to disengage" are displayed alternately on the touch panel.

- When you press the help key, the message "Please turn on power again" is displayed on the touch panel.

Solution

- Enter the correct HDD lock password.

- Check the installation status of EEPROM, and repair it when there is any abnormality.

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

- Contact the service manager of Konica Minolta.

Faulty part isolation DIPSW

DIPSW18-7

Control while detached

HDD unusable (HDD disconnected)

*When connected to IC-602, do not detach.

    - C-D0F1

Code

C-D0F1

Classification

Main body: Overall control EEPROM abnormality

Cause

EEPROM cannot be accessed normally

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Overall control board (OACB)

- EEPROM

Correction

Note

- The message "Please call service" is displayed on the panel.

Solution

- Check the installation status of EEPROM, and repair it when there is any abnormality.

- When the HDD lock password is disabled, replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

- When the HDD lock password is enabled, contact the service manager of KMBT.

Faulty part isolation DIPSW Control while detached

    - C-D0F5

Code

C-D0F5

Classification

Main body: File open error

Cause

RawHddMnger failed to open the file. The identification character "FILE" at the beginning of the data is detected.

* No SC Count

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

      - Overall control board (OACB)

      - Hard disk/1 (HDD1)

      - Hard disk/2 (HDD2)

      - Hard disk/3 (HDD3)

Correction

Note

      - The message "Please call service" is displayed on the panel.

Solution

- Check the connection between each HDD, and repair it when there is any abnormality.

- Format the HDD (Refer to I.4.15.2.Format HDD All Data)

- Replace HDD1, HDD2 and HDD3 (Refer to "I.4.7.8.Hard disk replacing procedure")

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW Control while detached

    - C-D0F6

Code

C-D0F6

Classification

Main body: File close error

Cause

RawHddMnger failed to close the file. The identification character "FILE" at the beginning of the data file is detected.

* SC is not counted.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

      - Overall control board (OACB)

      - Hard disk/1 (HDD1)

      - Hard disk/2 (HDD2)

      - Hard disk/3 (HDD3)

Correction

Note

      - The message "Please call service" is displayed on the panel.

Solution

- Check the connection between each HDD, and repair it when there is any abnormality.

- Format the HDD (Refer to I.4.15.2.Format HDD All Data)

- Replace HDD1, HDD2 and HDD3 (Refer to "I.4.7.8.Hard disk replacing procedure")

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW Control while detached

    - C-D0F8

Code

C-D0F8

Classification Main body: - Cause

Contact the service manager of KM.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

Correction

Note

      - The message "Please call service" is displayed on the panel.

Solution

1.

Faulty part isolation DIPSW Control while detached

    - C-E007

Code

C-E007

Classification

Main body: DIMM initialization abnormality

Cause

During the boot process, it is found that the DIMM/1 (DIMM1), DIMM/2 (DIMM2), DIMM/3 (DIMM3), DIMM/4 (DIMM4) is incorrectly installed on the board.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer image processing board (PRIPB)

- Overall control board (OACB)

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check that DIMM/1 (DIMM1), DIMM/2 (DIMM2), DIMM/3 (DIMM3), DIMM/4 (DIMM4) on the board are correctly installed, and repair it when there is any abnormality.

- Check the connector connection and the wiring of PRIPB and OACB, and repair it when there is any abnormality.

- Replace PRIPB.

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW Control while detached

    - C-E009

Code

C-E009

Classification

Main body: Paper interval processing error

Cause

The effective signal of the belt, the patch, and the registration is activated while the image effective signal of the normal image output or the test pattern output is ON.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

- Printer control board (PRCB)

- Printer image processing board (PRIPB)

- Overall control board (OACB)

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

- The message "Please turn on power again" is displayed on the touch panel.

Solution

- Check the connector connection and the wiring on PRCB, PRIPB and OACB and repair it when there is any abnormality.

- Replace PRCB.

- Replace PRIPB.

- Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW Control while detached

    - C-E012

Code

C-E012

Classification

Main body: -

Cause

"1" is selected on DIPSW49-1.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

•

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

      - The message "Please turn on power again" is displayed on the touch panel.

Solution

1. Select "0" on DIPSW49-1. Faulty part isolation DIPSW Control while detached

    - C-E013

Code

C-E013

Classification

Main body: -

Cause

"1" is selected on DIPSW49-1.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

•

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

      - The message "Please turn on power again" is displayed on the touch panel.

Solution

1. Select "0" on DIPSW49-1. Faulty part isolation DIPSW Control while detached

    - C-E014

Code

C-E014

Classification

Main body: -

Cause

"1" is selected on DIPSW49-1.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

•

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

      - The message "Please turn on power again" is displayed on the touch panel.

Solution

1. Select "0" on DIPSW49-1. Faulty part isolation DIPSW Control while detached

    - C-E015

Code

C-E015

Classification

Main body: -

Cause

"1" is selected on DIPSW49-1.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

•

Correction

Deactivate and activate the sub power switch (SW2) of the main body.

Note

      - The message "Please turn on power again" is displayed on the touch panel.

Solution

1. Select "0" on DIPSW49-1. Faulty part isolation DIPSW Control while detached

    - C-E017

Code

C-E017

Classification

Main body: ME disable error

Cause

BIOS of the overall control board (OACB) abnormality.

* No SC Count

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

      - Overall control board (OACB)

Correction

Note

      - The message "Please call service" is displayed on the panel.

Solution

1. Replace OACB (Refer to "G.3.2.8.Overall control board (OACB)")

Faulty part isolation DIPSW Control while detached

    - C-E018

Code

C-E018

Classification

Main body: Erratic pagination

Cause

An erratic pagination is detected on the job during printing.

Measures to take when an alert occurs

The main body and the option stop immediately to deactivate the main relay.

Estimated abnormal parts

•

Correction

Note

      - The message "Please call service" is displayed on the panel.

Solution

- Operate according to the procedures displayed on the touch panel.

- Deactivate the main power after you deactivate the sub power.

Faulty part isolation DIPSW Control while detached